Interface specification for 
WISTOM application sw api

Purpose

Specify the WISTOM V2 interface for the Linux based WEB2017 platform

Scope

Describe the API protocol and format and specify all available API commands in detail.

# Introduction

## Revision History

| Revision | Date | Made By | Change |
| --- | --- | --- | --- |
| A | 2002-09-06 | MHO | Converted from old document number structure. |
| B | 2003-03-05 | MHO | Major overhaul of API commands. Added TEST REGD, OPM# TPWR. Added new tags to TEST DRIV, OPM# CNFG, OPM# TRSH and OPM# MINL. |
| C | 2003-10-16 | JWI | Updated error causes. Added SMGR DUMP and SMGR CLRD. New format for PULF PULS. Added OPM# FILW to replace WICA FBDW. Moved side att calc to OPM# CNFG. |
| D | 2003-10-22 | JWI, MHO | Updated for R6 SW Added alarm types for OPM and temperature, and alarm time stamps. Added tags for pulse period in WICA CTEL. Added SMGR TEMP tags 3&4, OPM# CHNL/CHAL tags 12-14, deleted tag 3, OPM# FRQO, ALMH TITY, SMGR UPTI, SMGR LED#, SPEC SWIN, SPEC SWCO, SPEC SWMO. Added SMGR UNIN. SMGR HIDD tag 2. Added SPEC LMTS, TEST ALMR, TEST ALMS, TEST ALMU. Added tag 100 with the switch port to a lot of operations. The tag is optional (only included if number of inputs > 1). Increased max number of configured channels to 1024. Removed SMGR UNIN. Updated ALMH ALRM with new tags. Changed tag id for ALMH UNSU. Added chapter about alarm handling. |
| D1 | 2004-03-30 | MHO | SMGR UPGS tag 1 removed; SPEC SUBS wrong tag id should be 1 not 2; OPM# OSNR changed mandatory/optional for set/get operations. OPM OSNR changed max limit for tag 4. |
| E | 2005-01-25 | MHO, RTE, JWI | Fixed faulty alarm ID for temp alarm. Added SMGR INST and SMGR SCFG. Added SPEC COMP. Added tags to WICA PWRC. Added SPEC PWRA. Updated limit for LGIN UADD tag 4. Added tag 5 to OPM# PCRI. Added tag 27 to SPEC CHNL. RTE Added comments on SMGR INFO Added text about Min and Max chars in user and passwd data. Limited SMGR SER# to one port. Clarified that tag 100 is mandatory for OPM# CHNL and CALD in automatic mode. Added tag 6 to SMGR IP## Corrected typos and added extra information. |
| F | 2005-08-25 | MHO | Updated alarm new channel count, added new alarms system module status and system event.. Added SMGR API#. |
| G | 2005-12-13 | MHO JWI RTE | Added PULF RTEC , WICA CALD. Added alarm id 22 (unconfigured channel found), added alarm for system event from system manager. Updated switch handling for OPM# FSPC, TSPC, WSPC, RAWD and TPWR. Added SPEC CTBL. Added tag 5 to OPM# CHCO (channel auto-numbering) and updated OPM# CHNL, CALA, CALD, CHAL. Changed System event status code “Save to Flash operation” to 4. Added INFO tag 54 “Switch revision” Added CHCO TAG 4 “Search free peaks” Changed limits on OPM# TRSH, PCRI |
| H | 2006-02-20 2006-06-07 | JWI MHO | Added OPM# CSPC. Added SMGR SATR, SMGR SDTR, SMGR SLTR. |
| J | 2007-04-13 | MHO | Added tag 6 in OPM# PCRI. PULF REGC tag 5 UINT8 PULF REGC tag 4 and 5 changed description Added tag 3 to PULF RESC. Added System manger to System module status (alarm). Added tag 5 in WICA PWRC. |
| K | 2007-08-22 | MHO | Converted to new Proximion template. Added OPM# SFBW. Added SMGR INST tag 2 |
| L | 2020-12-31 | KFr | Updates for WEB-2017 software and hardware. APP WSNS, SPEC FILT more ports. |
| M.1 | 2024-08-05 | KFr | WICA SABC new function, WSNS PARA tag 9 new (dispersion compensation) |
| M4 | 2025-02-19 | KFr | WSNS PARA tag 10 new (dispersion compensation 3rdOrder) and 11 (Second order fit of sensor peaks) |

## Related Documents

0002019  Wistom User Guide

## Acronyms and Abbreviations

| Acronym/ Abbreviation | Description |
| --- | --- |
| API | Application programmers interface |
| N/A | Not applicable |

# Table of Contents

1	Introduction	2

1.1	Revision History	2

1.2	Distribution	4

1.3	Distribution	5

1.4	Related Documents	5

1.5	Acronyms and Abbreviations	5

1.6	Definitions	5

2	Table of Contents	6

3	API	7

3.1	Overview	7

3.2	Design	7

4	User levels and other restrictions	15

5	API command set	15

5.1	General	15

5.2	Command Overview	15

5.3	Alarms	19

5.4	Error Causes	21

5.5	Detailed command descriptions	22

6	References	89

# API

## Old and new hardware compatibiity

This manual is valid for the new, Linux based, hardware, WEB2017, it is also mostly valid for the  old hardware platform WEB2007 as the purpose has been to retain backward compatibility. However, there are differences and it is recommended to use revision K of this document when information is needed for WEB2002. There are also difference as WEB2017 has a 8 times higher sampling rate, higher ADC resolution and support more ports.

## Overview

This document has been updated with the modification included for the WistSense app and the updates from the switch to a new hardware platform WEB-2017 using a Linux instead of OSE based platform

The API consists of the commands and responses outlined in Table 1. More detailed descriptions can be found in section 3.2.

Table 1	API overview

| Command | Response | Description |
| --- | --- | --- |
| Login | LoginResult | Starts a session. The command holds user id and password. The response holds the user access rights or an error code. |
| Logout | - | This ends a session. There is no response since all instances immediately terminates. |
| Get | GetRes/GetErr | Get a setting, a value, or data from WISTOM. |
| Set | SetAck/setNAck | Set a setting or a value in WISTOM. This could also be a subscription for alarms or data, or it could trigger an operation. |
| (get) | Alarm | Alarm message, is sent spontaneously (after subscription) or triggered by a get request |

## Design

### General

The Wistom Application Programmers Interface (API) has been developed to give a generic and versatile interface to all Wistom functions. The API provides a rather small command set that still lets the user read and write almost any value in Wistom. Furthermore, user identities and passwords are used to provide the necessary security features.

The API exists in two forms; either as a text-based command set ( the MMI) or as a binary protocol that can be transferred on any interface (e.g. TCP/IP). The binary representation is described in section 3.2.6.

### Begin and End a Session

The first command in an API session must always be a login. The command shall be followed with a user identity and a password. User ids are stored in Wistom with the current password and also the user’s privileges. So a login is compared to the internally stored settings, and the API session is configured correspondingly.

The user privileges are defined with a user level (a number from 1 to 5), and each API operation can only be executed or read by a user with a user level of a certain value (or less).

A session is ended with the logout command (with no parameters). There is no response to the logout command.

### Fundamental Operations

The core of the API is just two commands. A get is used to fetch any parameter or data from Wistom, and a set is used to set a parameter, trigger a function or perform an operation. The commands are used to access defined operations in Wistom, which are identified with an application id (app) and an operation id (op).  Each app/op pair gives the user access to one or more parameters, functions or data.

The normal responses to get and set are “get result” (getRes) and “set acknowledge” (setAck) respectively. If something went wrong with the get or set, “get error” (getErr) or “set negative acknowledge” (setNAck) is returned instead.

Since multiple get and set commands can be issued simultaneously or in a tight sequence, each command is accompanied with a token. The token is an integer value, which is always returned in the response from Wistom. It is up to the user to employ the tokens wisely, so that it can discern one operation from the other.

### Data Format

The data part of an API message is structured in a special way. Since API messages can be both text-based and binary, it is essential to be able to discern how the data is structured, i.e. if the data is composed of integers, floats or just plain text strings. Furthermore, the amount of data for a specific operation may vary with the application, and over time. The selected solution to this has been to “tag” each data element. For each application/operation pair a set of tags is defined. Each tag defines exactly how much data that follows and how it shall be interpreted (e.g. five unsigned integers). The amount of tags and data varies depending on what the user wants to achieve.

The data types that are used in the Wistom API are listed in Table 2.

Table 2: API data types.

| Type | Description |
| --- | --- |
| INT8, INT16, INT32 | Signed integer. 8, 16 and 32 denotes the number of bits in the integer. E.g. INT32 for a 32-bit signed integer. |
| UINT8, UINT16, UINT32 | Unsigned integer. 8, 16 and 32 denotes the number of bits in the integer. E.g. UINT16 for a 16-bit signed integer. |
| FLOAT32, FLOAT64 | Floating-point values. 32 and 64 bits long respectively. Floating-point values are according to IEEE 754. |
| STRING | A null-terminated ASCII string. |

### Examples

The following examples may further clarify the above discussion on applications, operations, tags and data:

Example 1:  Get board temperature

get SMGR TEMP   
 getRes SMGR TEMP 1 27.3 2 32.5 3 45.1

The command is a get from the application System Manager (“SMGR”), with the operation Temperature (“TEMP”). The response to this is a getRes with the tags “1” and “2”. Both tags are followed by a 32-bit floating-point value that denotes the board temperature and the sensor temperature respectively.

Example 2: Set channel configuration

set SPEC CHNL 2 1 3 1 4 194.1e12 12 25 8 0.001e12  
 setAck

The command is a set from the application Spectrum Parameters (“SPEC”), with the operation Channel Configuration (“CHNL”) with the tags “2”, “3”, “4”, “12” and “8”. This command will set the channel configuration for channel 1  (tag “2”) to “configured” (tag “3”) with the center frequency 194.1 THz (tag “4”), OSNR warning level to 25 dB (tag “12”) and with the frequency hysteresis 1 GHz (tag “8”). See section 5.5.1 for more information about the channel configuration. Note that channel parameters that are not included in this command are unchanged.

### Binary Representation

The Wistom API is by definition in big-endian format, i.e. the most significant byte shall be sent first for values that consist of more than one byte. This means that for example on a desktop PC, all data must be switched to little-endian format before it can be readable by a program.

The data field tags are 8-bit integers and the type of the actual data varies but is always defined by the tag value.

How the API commands shall be constructed in a binary protocol (e.g. TCP/IP) is described in the tables below.

Table 3: API login

| Element | Type | Description |
| --- | --- | --- |
| 0x0001 | UINT16 | The login command identifier. |
| Token | UINT16 | A value that is returned in the loginRes. |
| App | UINT32 | Four ASCII characters, denoting the application (see section 3.2.3). This must be set to “LGIN” if using revision 2 of the API. |
| Op | UINT32 | Four ASCII characters, denoting the operation (see section 3.2.3). This must be set to “API2” if using revision 2 of the API. |
| Datalen | UINT32 | Number of bytes following this field (including the null-terminators in the strings). |
| UserId | STRING | A null-terminated string with the username. |
| Passwd | STRING | A null-terminated string with the password. |

Table 4: API loginRes

| Element | Type | Description |
| --- | --- | --- |
| 0x0101 | UINT16 | The loginRes command identifier. |
| Token | UINT16 | Same value that was received in the get command. |
| App | UINT32 | Same value that was received in the set command. |
| Op | UINT32 | Same value that was received in the set command. |
| Datalen | UINT32 | Number of bytes following this field. Always set to 4. |
| Result | UINT32 | If the login succeeded the user level is returned, else an internal error code. |

Table 5: API logout

| Element | Type | Description |
| --- | --- | --- |
| 0x0002 | UINT16 | The logout command identifier. |
| Token | UINT16 | No response to this command, so this is unused. |
| Dummy1 | UINT32 | Dummy for consistency with other commands. |
| Dummy2 | UINT32 | Dummy for consistency with other commands. |
| Datalen | UINT32 | Number of bytes following this field (including the null-terminators in the strings). |
| UserId | STRING | A null-terminated string with the user name. |

Table 6: API set

| Element | Type | Description |
| --- | --- | --- |
| 0x0003 | UINT16 | The set command identifier. |
| Token | UINT16 | A value that is returned in the setAck/setNAck. |
| App | UINT32 | Four ASCII characters, denoting the application (see section 3.2.3). |
| Op | UINT32 | Four ASCII characters, denoting the operation (see section 3.2.3). |
| datalen | UINT32 | Number of bytes following this field. |
| Data |  | Tags and data as defined in section 3.2.4. |

Table 7: API setAck

| Element | Type | Description |
| --- | --- | --- |
| 0x0103 | UINT16 | The setAck command identifier. |
| Token | UINT16 | Same value that was received in the set command. |
| App | UINT32 | Same value that was received in the set command. |
| Op | UINT32 | Same value that was received in the set command. |
| Datalen | UINT32 | Always 0. |

Table 8: API setNAck

| Element | Type | Description |
| --- | --- | --- |
| 0x0203 | UINT16 | The set command identifier. |
| Token | UINT16 | Same value that was received in the set command. |
| App | UINT32 | Same value that was received in the set command. |
| Op | UINT32 | Same value that was received in the set command. |
| Datalen | UINT32 | Always set to 4 |
| cause | UINT32 | Error cause |

Table 9: API get

| Element | Type | Description |
| --- | --- | --- |
| 0x0004 | UINT16 | The get command identifier. |
| Token | UINT16 | A value that is returned in the getRes/getErr. |
| App | UINT32 | Four ASCII characters, denoting the application (see section 3.2.3). |
| Op | UINT32 | Four ASCII characters, denoting the operation (see section 3.2.3). |
| datalen | UINT32 | Number of bytes following this field. |
| Data |  | Tags and data as defined in section 3.2.4. |

Table 10: API getRes

| Element | Type | Description |
| --- | --- | --- |
| 0x0104 | UINT16 | The getRes command identifier. |
| Token | UINT16 | Same value that was received in the get command. |
| App | UINT32 | Same value that was received in the get command. |
| Op | UINT32 | Same value that was received in the get command. |
| Datalen | UINT32 | Number of bytes following this field. |
| Data |  | Tags and data as defined in section 3.2.4. |

Table 11: API getErr

| Element | Type | Description |
| --- | --- | --- |
| 0x0204 | UINT16 | The getErr command identifier. |
| Token | UINT16 | Same value that was received in the get command. |
| App | UINT32 | Same value that was received in the get command. |
| Op | UINT32 | Same value that was received in the get command. |
| Datalen | UINT32 | Always set to 4 |
| cause | UINT32 | Error cause |

Table 12: API Alarm, type 0 (no time stamp)

| Element | Type | Description |
| --- | --- | --- |
| 0x0105 | UINT16 | The spontaneous alarm command identifier. |
| token | UINT16 | Same value that was received in the subscription request. |
| length | UINT32 | Number of bytes following this field. The following data may be repeated if more than one alarm is sent at once. |
| alarm Id | UINT16 | The alarm identifier. See section 5.3. |
| alarm subId | UINT16 | The alarm sub-identifier. See section 5.3. |
| status | UINT32 | Alarm status. See section 5.3. |

Table 13: API Alarm, type 1 (SNMP style). Default.

| Element | Type | Description |
| --- | --- | --- |
| 0x1105 | UINT16 | The spontaneous alarm command identifier. |
| token | UINT16 | Same value that was received in the subscription request. |
| length | UINT32 | Number of bytes following this field. The following data may be repeated if more than one alarm is sent at once. |
| alarm Id | UINT16 | The alarm identifier. See section 5.3. |
| alarm subId | UINT16 | The alarm sub-identifier. See section 5.3. |
| status | UINT32 | Alarm status. See section 5.3. |
| time stamp | UINT32 | Number of 10-milliseconds ticks since system start. |

Table 14: API Alarm, type 2 (Unix style).

| Element | Type | Description |
| --- | --- | --- |
| 0x2105 | UINT16 | The spontaneous alarm command identifier. |
| token | UINT16 | Same value that was received in the subscription request. |
| length | UINT32 | Number of bytes following this field. The following data may be repeated if more than one alarm is sent at once. |
| alarm Id | UINT16 | The alarm identifier. See section 5.3. |
| alarm subId | UINT16 | The alarm sub-identifier. See section 5.3. |
| status | UINT32 | Alarm status. See section 5.3. |
| time stamp | UINT32 | Seconds since 1970-01-01 00:00. |

Table 15: API Alarm, type 3 (uptime in millisecond ticks).

| Element | Type | Description |
| --- | --- | --- |
| 0x3105 | UINT16 | The spontaneous alarm command identifier. |
| token | UINT16 | Same value that was received in the subscription request. |
| length | UINT32 | Number of bytes following this field. The following data may be repeated if more than one alarm is sent at once. |
| alarm Id | UINT16 | The alarm identifier. See section 5.3. |
| alarm subId | UINT16 | The alarm sub-identifier. See section 5.3. |
| status | UINT32 | Alarm status. See section 5.3. |
| time stamp | UINT32 | Uptime in millisecond ticks. |

Table 16: API Alarm, type 4 (Unix style in milliseconds).

| Element | Type | Description |
| --- | --- | --- |
| 0x4105 | UINT16 | The spontaneous alarm command identifier. |
| token | UINT16 | Same value that was received in the subscription request. |
| length | UINT32 | Number of bytes following this field. The following data may be repeated if more than one alarm is sent at once. |
| alarm Id | UINT16 | The alarm identifier. See section 5.3. |
| alarm subId | UINT16 | The alarm sub-identifier. See section 5.3. |
| status | UINT32 | Alarm status. See section 5.3. |
| time stamp 1 | UINT32 | Seconds since 1970-01-01 00:00. |
| time stamp 2 | UINT16 | Milliseconds (0-999). |

For specifications of the individual alarms and sub alarms, see section 5.3 Alarm.

# User levels and other restrictions

Each user id is configured with a “user level”. This means that he will only have access to the operations that are allowed for that level. Table 17 lists and describes all user levels.

Table 17	User levels

| User level | Description |
| --- | --- |
| 5 | Read-only access. This user can only read information that relates to the basic functions of the product. He cannot modify anything inside the product and cannot make “subscriptions” to alarms or data. |
| 4 | Intermediate monitoring access. Only minor configuration changes available. |
| 3 | Application configuration rights. This user can configure the applications within the system. E.g. set up the OCM channels, define OSNR. |
| 2 | Can change most parameters, perform SW upgrades etc. Minimum user level for customers. |
| 1 | “Root access”. Users with this level can do anything with the system. Normally these users are restricted to login to only one defined interface (for security reasons). Proximion personnel only. |

# API command set

## General

The following sections show all available get and set application/operation pairs that are available in WISTOM. There is one section for each “application”, and in that section, all operations that are available for it are defined.

## Command Overview

| Application | Operation | Description |
| --- | --- | --- |
| SPEC | CHNL | Channel configuration. Wavelengths, power, alarm levels etc. |
| SPEC | CTBL | Configured channels table |
| SPEC | COMP | Spectrum compression. |
| SPEC | DELC | Delete Channel configuration. |
| SPEC | FILT | Hardware filtering of spectrum |
| SPEC | LMTS | Calibration limits. |
| SPEC | PWRA | Switch power adjustment. |
| SPEC | SUBS | Sub-scanning. (Emulated in WEB-2017) |
| SPEC | SWCO | Switch port configuration |
| SPEC | SWIN | Switch install |
| SPEC | SWMO | Switch scan mode |
| WICA | ACUT | Actuator configuration update time (Not used) |
| WICA | BIAS | Bias offset on ADC channel |
| WICA | CHAN | Which optical ADC-channel to use |
| WICA | CTEL | LUT indexing configuration. |
| WICA | CALD | Calibration daemon (not used) |
| WICA | FBDW | WISTOM filter bandwidth |
| WICA | FLIM | Calibrated spec limits. |
| WICA | FRQC | Frequency compensation |
| WICA | LUT# | A/D-converter look-up table (No function in WEB2017) |
| WICA | LUTF | Factor LUT used in calibration |
| WICA | LUTK | Factor used in frequency calibration. |
| WICA | LUTL | Offset LUT used in frequency calibration. |
| WICA | LUTP | LUT for pulse period. |
| WICA | PABC | Pulse amplifier attenuation control |
| WICA | PULF | Actuator pulse form |
| WICA | PULP | Pulse period |
| WICA | PWRC | Power compensation |
| WICA | SABC | Sampling auto bias control |
| WICA | SCAS | Scan Size (Read only in WEB2017?) |
| WICA | STAB | Save tables to permanent storage |
| WICA | STRD | Start delay |
| WICA | TLIM | Calibrated temp limits. |
| SMGR | API# | API definition table |
| SMGR | FLSH | Save configuration |
| SMGR | HIDD | Activate some hidden MMI commands |
| SMGR | INFO | Product info |
| SMGR | IP## | IP settings |
| SMGR | INST | Feature installation |
| SMGR | LED# | LED status (WEB-2017 not implemented) |
| SMGR | REST | Restart system |
| SMGR | SATR | SNMP Add trap receiver |
| SMGR | SCFG | SNMP configuration |
| SMGR | SDTR | SNMP Delete trap receiver |
| SMGR | SER# | Serial (RS232) settings |
| SMGR | SLTR | SNMP List trap receivers |
| SMGR | TEMP | Temperature |
| SMGR | TIME | Date and time |
| SMGR | UPTI | System running time |
| SMGR | VOLT | Returns system voltages and current |
| LGIN | COPW | Change others password |
| LGIN | CPWD | Change password |
| LGIN | SINF | Session info |
| LGIN | UADD | User add |
| LGIN | UDEL | User delete |
| LGIN | UINF | User information |
| OPM# | AVRG | OPM average |
| OPM# | CALA | OPM calibration data for all channels |
| OPM# | CALC | OPM power calculation |
| OPM# | CALD | OPM channel status for calibration |
| OPM# | CHAL | OPM all channel status |
| OPM# | CHCO | OPM channel configuration |
| OPM# | CHNL | OPM channel status |
| OPM# | CNFG | OPM configuration |
| OPM# | CSPC | OPM compact spectrum |
| OPM# | ENAB | OPM enable |
| OPM# | FILW | OPM filter width |
| OPM# | FRQO | OPM frequency options |
| OPM# | FSPC | OPM frequency spectrum |
| OPM# | MINL | OPM min. level |
| OPM# | OSNR | OPM OSNR configuration |
| OPM# | OUTP | OPM output spectrum specification |
| OPM# | PCRI | OPM peak criteria |
| OPM# | RAWD | OPM raw data |
| OPM# | SFBW | OPM system filter bandwidth |
| OPM# | SWHA | OPM Switch handling |
| OPM# | TPWR | OPM total power over spectrum |
| OPM# | TRSH | OPM threshold |
| OPM# | TSPC | OPM time spectrum |
| OPM# | WSPC | OPM wavelength spectrum |
| PULF | COMP | Pulse frequency control |
| PULF | PULS | Pulse resonance spectrum (with fractional part) |
| PULF | REGZ | Frequency Z regulator |
| PULF | REGV | Pulse frequency controller values |
| PULF | RESC | Pulse resonance spectrum configuration |
| PULF | REGC | Frequency regulator control |
| PULF | RTEC | Repeat time event ctrl |
| ALMH | ALRM | Alarm status |
| ALMH | SUBS | Alarm subscribe |
| ALMH | TITY | Alarm time stamp type |
| ALMH | UNSU | Alarm unsubscribe |
| WSNS | NEXT | Get current peak frequencies. |
| WSNS | PARA | Parameters in wistsens. |
| WSNS | PORT | Calibration and sensor ports. |
| WSNS | ENAB | Enable/disable wistsens. |
| WSNS | DATA | Get spectra for debugging. |
| WSNS | RAWB | Returns raw spectrum without bias correction and debug info |
| WSNS | FILT | Software based filtering of spectra |

## Alarms

### General

There are two ways to obtain alarms generated by the system, either by polling the system or by subscribing to alarms.

Polling the system is accomplished by using the API command get ALMH ALRM. This request returns current alarm status.

If subscribing to an alarm, an API message is sent to the subscriber when a status change occurs. It is important to understand how this feature is handled by the system in order to set up and maintain a subscription.

To set up a subscription use ALMH SUBS and to terminate subscriptions use ALMH UNSU.

The most important aspect to remember is that a subscription is associated with a particular API session. Therefore, a subscription only lives as long as the API session lives. Also, it is only possible to receive an alarm message and maintain current subscriptions in the system from where the actual subscription was made (API session).

An alarm token of choice is given to each subscription (this is not the same as the API message token). The alarm token has a dual purpose. First of all, it is returned in all subsequent API alarm messages. Secondly, it is used as an identifier when terminating a subscription or bundled subscriptions. By giving several subscriptions the same token, the subscriptions will be bundled together and can be unsubscribed with one operation.

NOTE! Since a subscription is associated with an API session it is possible for two different subscriptions from different API sessions to have the same token without any risk of interference.

There are four different API alarm messages. Depending on what time format is used (ALMH TITY), the corresponding message is returned.

NOTE! If the time format is changed all subsequent API alarm messages will also change.

### Alarm Specification

Implemented alarms are described in detail in table below.

| Alarm type | ID | Sub ID | Alarm status |
| --- | --- | --- | --- |
| OPM channel status | 20 | Channel ID 1 – 1024. | 0x00PPFFOO PP: Power FF: Frequency OO: OSNR OK 01 NOT FOUND 04 WARNING_LOW 10 WARNING_HIGH 20 ALARM_LOW 40 ALARM_HIGH 80 |
| OPM new channel count | 21 | Switch Port ID 1 – 16 | <Number of OPM channels per Switch Port> Automatic mode: The number of channels detected by OPM. Configured mode: The number of configured channels that have other status than “missing”. |
| OPM unconfigured channel found | 22 | Index (1-1024) | OPM reports the frequency of a found channel that is not included in the channel configuration. The index is an arbitrary number that separates different unconfigured channels from each other. Bits 18-21: < Switch Port ID 1 – 16> Bits 0-17: <Frequency [GHz] as an integer> This alarm only applies when OPM works in configured mode and when OPM# CHCO tag 4 is set to 1. |
| Temperature status | 30 | 1 (calibrated range) | Sensor temperature status (compared to SMGR INFO tags 82 and 83). OK 0x00000001 Too low 0x00000040 Too high 0x00000080 |
| Temperature status | 30 | 2 (configured range) | Sensor temperature status (compared to SMGR TEMP). OK 0x00000001 Too low 0x00000040 Too high 0x00000080 |
| System event | 90 | Event: | System event status: |
|  |  | 1 Error Handler | Error dump created 10 |
|  |  | 2 System manager | XXXX NOT IMPLEMENTED XXXX Save to Flash operation 4 Software upgrade started 10 Software upgrade ended 11 Software upgrade aborted 12 (software upgrade is defined either as a TFTP upgrade or directly through the API) |
| System module status | 91 | Module: | Module status: |
|  |  | 1 Switch | OK 0x00000001 Not responding 0x00000008 HW comm. read timeout 0x00000100 HW comm. write timeout 0x00000200 HW comm. read error 0x00000400 HW comm. write error 0x00000800 |
|  |  | 2 System manager | OK 0x00000001 Unstable 0x00001000 |

## Error Causes

NOTE: the following only applies to API version 2.

This table specifies the error causes that can be returned with a setNAck or getErr command.

The error cause is divided in two parts. The lower 16bits are the cause code and the higher 16bits specifies the extra error data.

Table 18	Set and get error causes

| Cause | Extra data | Description | Comment |
| --- | --- | --- | --- |
| 0 |  | No error |  |
| 1 |  | Unknown |  |
| 2 |  | No response | Internal timeout. |
| 3 |  | Request failed | Typically due to a configuration mismatch. |
| 4 |  | Illegal application |  |
| 5 |  | Illegal operation |  |
| 6 | Tag number | Illegal tag |  |
| 7 |  | Mandatory tags missing |  |
| 8 | Tag number | Tag value too low |  |
| 9 | Tag number | Tag value too high |  |
| 10 |  | Invalid value |  |
| 11 |  | Length mismatch |  |
| 12 |  | Not initialized |  |
| 13 |  | Invalid configuration | Typically a channel configuration that cannot be used by OCM or OPM. |
| 14 |  | Out of resources | There are for instance a limited number of available alarm subscribers. |
| 15 |  | User level restriction |  |
| 16 |  | Operation not available for “get” |  |
| 17 |  | Operation not available for “set” |  |

This table specifies the result codes returned in loginRes.

Table 19	Login result codes

| Result | Description |
| --- | --- |
| 0x00000001 | Logged in with user level 1 |
| 0x00000002 | Logged in with user level 2 |
| 0x00000003 | Logged in with user level 3 |
| 0x00000004 | Logged in with user level 4 |
| 0x00000005 | Logged in with user level 5 |
| 0x10000000 | Max number of users logged in |
| 0x10000001 | Max number of users in system reached |
| 0x10000002 | Max number of login attempts |
| 0x10000003 | Permission denied |
| 0x10000004 | Not allowed from this interface |
| 0x10000005 | Illegal interface type (internal) |
| 0x10000006 | Time lock active (internal) |
| 0x10000007 | Wrong password |
| 0x10000008 | Unknown user name |

## Detailed command descriptions

The table columns have the following meanings:

Application	The application that this table references. Same as the section header. The application code is the code or value that is used to reference this application.

Operation	The name of the operation that is described in this table row(s). The operation code is the code or value that is used to reference this application.

Max. User Lvl	The maximum allowed user level (section 4) that is required to “get” or “set” this operation. A “-“ (minus sign) means that the operation is not allowed for this command.

Data Field	The data field consists of “tags” with corresponding values. The tags defines what kind of data follows, and its type.

Data Tag	The tags are 8-bit values that are unique within this operation.

Data type	The type of the data item. E.g. 32-bit integer, four characters, a zero-terminated ASCII string etc.

Class	Whether this data item is mandatory (M) or optional (O). This is specified independently for “get”, “set” and “getRes” commands

### SPEC: Spectrum Parameters

| Application | App code (ApiApp_t) |
| --- | --- |
| Spectrum Parameters | SPEC |
| Operation | Op code (ApiOp_t) | Max. User Lvl |  | Data Field | Tag | Data type | Class |  |  | Description |
|  |  | Get | Set |  |  |  | set | get | res |  |
| Channel configuration | CHNL | 5 | 3 | ChannelIdMap | 1 | 32*UINT32 (1024 bits) | M1 | - | - | Identifies what channels this operation relates to. This is a bitmap where a set bit means that this operation refers to the channel with the number of the bit. |
|  |  |  |  | ChannelId | 2 | UINT16 | M1 | M | M | Alternative to ChannelIdMap. This is a value that specifies one specific channel. The value is the same as the number of the bit in ChannelIdMap, i.e. 1 – 1024. |
|  |  |  |  | SwitchPort | 100 | UINT8 | O | - | O | The switch port that this channel is valid for. This channel is only sent in the result if the number of inputs is more than one (1). Internally, any set value is ignored in that case. |
|  |  |  |  | ActivateMask | 3 | UINT16 | O | - | M | A bit mask where each bit corresponds to whether this channel is used for that application. Bit 0: Configured Bit 1: OCM Bit 2: OPM |
|  |  |  |  | FreqC | 4 | FLOAT64 (Hz) | O | - | M | Nominal frequency |
|  |  |  |  | PowerC | 5 | FLOAT64 (W) | O | - | M | Nominal power |
|  |  |  |  | Obsolete | 6 |  | - | - | - | Use of this tag results in SET_NACK/GET_ERR. |
|  |  |  |  | Obsolete | 7 |  | - | - | - | Use of this tag results in SET_NACK/GET_ERR. |
|  |  |  |  | FreqHysteresis | 8 | FLOAT64 (Hz) | O | - | M | How much hysteresis that is imposed on frequency-related warning and error levels. |
|  |  |  |  | Obsolete | 9 |  | - | - | - | Use of this tag results in SET_NACK/GET_ERR. |
|  |  |  |  | Obsolete | 10 |  | - | - | - | Use of this tag results in SET_NACK/GET_ERR. |
|  |  |  |  | Obsolete | 11 |  | - | - | - | Use of this tag results in SET_NACK/GET_ERR. |
|  |  |  |  | OsnrWarn | 12 | FLOAT64 (dB) | O | - | M | OSNR (Optical Signal to Noise Ratio) where a warning is issued. |
|  |  |  |  | OsnrAlm | 13 | FLOAT64 (dB) | O | - | M | OSNR (Optical Signal to Noise Ratio) where an alarm is issued. |
|  |  |  |  | OsnrHysteresis | 14 | FLOAT64 (dB) | O | - | M | How much hysteresis that is imposed on OSNR-related warning and error levels. |
|  |  |  |  | DeltaFreqOsnr | 15 | FLOAT64 (Hz) | O | - | M | How far (frequency-wise) from FreqC the noise is measured when calculating OSNR. |
|  |  |  |  | OPMWindow | 16 | FLOAT64 (Hz) | O | - | M | This is how far from the central frequency that OPM searches for this channel. I.e. OPM looks in a window that is 2* OPMWindow Hz, centered around FreqC. |
|  |  |  |  | DeltaPower WarnHigh | 17 | FLOAT64 (dB) | O | - | M | How much the power can change before a warning is issued (relative to PowerC). |
|  |  |  |  | DeltaPower WarnLow | 18 | FLOAT64 (dB) | O | - | M | How much the power can change before a warning is issued (relative to PowerC). |
|  |  |  |  | DeltaPower AlmHigh | 19 | FLOAT64 (dB) | O | - | M | How much the power can change before an alarm is issued (relative to PowerC). |
|  |  |  |  | DeltaPower AlmLow | 20 | FLOAT64 (dB) | O | - | M | How much the power can change before an alarm is issued (relative to PowerC). |
|  |  |  |  | PowerHysteresis | 21 | FLOAT64 (dB) | O | - | M | How much hysteresis that is imposed on power-related warning and error levels. |
|  |  |  |  | DeltaFreq WarnHigh | 22 | FLOAT64 (Hz) | O | - | M | How much the channel can move in frequency before a warning is issued (relative to FreqC). |
|  |  |  |  | DeltaFreq WarnLow | 23 | FLOAT64 (Hz) | O | - | M | How much the channel can move in frequency before a warning is issued (relative to FreqC). |
|  |  |  |  | DeltaFreq AlmHigh | 24 | FLOAT64 (Hz) | O | - | M | How much the channel can move in frequency before an alarm is issued (relative to FreqC). |
|  |  |  |  | DeltaFreq AlmLow | 25 | FLOAT64 (Hz) | O | - | M | How much the channel can move in frequency before an alarm is issued (relative to FreqC). |
|  |  |  |  | Integration interval | 26 | FLOAT64 (Hz) | O | - | M | The calculation of the channel area is done by integrating over 2 times this interval. |
|  |  |  |  | Channel description | 27 | String | O | - | M | Channel description. Null terminated string with a max length of 20 characters. |
| Delete Channel Configuration | DELC | - | 3 | ChannelId | 2 | UINT16 | M | - | - | This value specifies the channel configuration to be deleted. The value is the same as the number of the bit in ChannelIdMap, i.e. 1 - 1024. |
| Configured channels table | CTBL | 5 | - | Num channels | 1 | UINT16 | - | - | M | The number of channels that are configured, i.e. channels where bit 0 of the activated mask is 1. |
|  |  |  |  | Channel table | 2 | <num channels> * UINT16 | - | - | M | List of configured channels. The channel numbers (1-1024) are reported as a sequence. |
| Calibration limits | LMTS | 4 | - | Start of calib spec | 1 | FLOAT64 (Hz) | - | - | M | Start frequency of calibrated spectrum |
|  |  |  |  | End of calib spec | 2 | FLOAT64 (Hz) | - | - | M | Stop frequency of calibrated spectrum |
|  |  |  |  | Start temp | 3 | FLOAT32 (Celsius ) | - | - | M | Start temperature of calibration |
|  |  |  |  | Stop temp | 4 | FLOAT32 (Celsius) | - | - | M | Stop temperature of calibration |
| Sub-scanning | SUBS | 4 | - | Sub scans | 1 | UINT8 | M | - | M | This will reduce the number of samples used in the system by binning the data. A value of 8 will give the full 14336 sample resolution, no effect of update frequency but some analysis can be quicker with lower values of 4,2 or 1. |
| Spectrum compression | COMP | 5 | 3 | Level | 1 | UINT8 | M | - | M | Obsolete: Might work.It is possible to compress spectrum data by averaging and concatenating consecutive samples. This will result in a spectrum with more sparse samples that will improve data processing and communication speed. Range 2n, where n=0-3 (20 = no compression). |
| Power adjustment. | PWRA | 5 | 3 | Switch port 1-50 (only installed ports) | 1-50 | FLOAT32 (dB). | O | - | O | Adjustment factor for switch port. The port is the same as the tag number. The tag is only reported if the switch port is installed. |
| Switch installation | SWIN | 5 | 1 | Port # | 1 - 50 | UINT8 | O | - | M | =0 Not available =1 Available |
|  |  |  |  | First switch setting (typical master) | 101 | UINT8*50 |  |  |  | How the first switch (typical master) should be set for all configured ports: Allowed value are 0-16 where 0 indicate that the switch is not used for that port. |
|  |  |  |  | Second switch setting (subswitch) | 102 | UINT8*50 |  |  |  | How the second switch (typical slave) should be set for all configured ports: Allowed value are 0-16 where 0 indicate that the switch is not used for that port. |
|  |  |  |  | Third switch setting | 103 | UINT8*50 |  |  |  | How the third switch (typical slave) should be set for all configured ports: Allowed value are 0-16 where 0 indicate that the switch is not used for that port. |
|  |  |  |  | Fourth switch setting | 104 | UINT8*50 |  |  |  | How the fourth switch (Not tested) should be set for all configured ports: Allowed value are 0-16 where 0 indicate that the switch is not used for that port. |
| Switch port configuration | SWCO | 5 | 3 | Port scan priority | 1-50 | UINT8 (0-100) | O | - | O | In automatic mode, the ports are scanned with this interval. Scan interval = port prio / ∑priorities [%] Prio = 0 -> Manual only The tag is only reported if the switch port is installed. |
|  |  |  |  | Port description | 51-100 | String | O | - | O | Switch port description. Null-terminated string with a max length of 20 characters. The tag is only reported if the switch port is installed. |
|  |  |  |  | Scan percentage | 101-150 | UINT8 (%) | - | - | O | 100 * port prio / ∑priorities [%] The tag is only reported if the switch port is installed. |
|  |  |  |  | Scan interval | 151-200 | FLOAT32 (s) | - | - | O | Acquisition time / (scan percentage/100). Acquisition time: sub scans * averages * 80 µs The tag is only reported if the switch port is installed. |
| Switch scan mode | SWMO | 5 | 3 | Mode | 1 | UINT8 | O | - | M | =0 Manual (the port indicated in tag 2 is scanned next) =1 Automatic mode (ports are scanned according to SPEC SWCO). |
|  |  |  |  | Port # | 2 | UINT8 | O | - | M | The port that is scanned in manual mode. |

### WICA: WISTOM Calibration

| Application | App code (ApiApp_t) |
| --- | --- |
| WISTOM Calibration | “WICA” |
| Operation | Op code (ApiOp_t) | Max. User Lvl |  | Data Field | Tag | Data type | Class |  |  | Description |
|  |  | Get | Set |  |  |  | set | get | res |  |
| Actuator pulse form | “PULF” | 1 | 1 | Pulse form as a |  | 16383*UINT8 (With size in header) |  |  |  | WEB2017 only: This is a number of values defining the pulse form. Only valid for new hardware (WEB-2017). Time step is 0.625 ns. Note that for pulse period evenly dividable by 16, only every 16 value is used, for evenly dividable by 8, only every 8 and so on. The pulse shape should be low pass filtered to reject frequencies above 50 MHz. |
| A/D-converter look-up table | “LUT#” | 1 | 1 | LUT | 1 | 4096 * UINT16 | M | - | M | No function, retained for backward compatibility |
| Power compensation | “PWRC” | 1 | 1 | P0 | 1 | FLOAT32 (W) | O | - | M | Constant value in linear approximation. P = (kP * x + P0) * ksw Where x is the sample value. |
|  |  |  |  | KP | 2 | FLOAT32 (W/sample unit) | O | - | M | Factor in linear approximation |
|  |  |  |  | TimeToStart | 3 | FLOAT64 (s) | O | - | M | Where on the fiber the LUT is valid |
|  |  |  |  | deltaTime | 4 | FLOAT64 (s) | O | - | M | Time step in lut. |
|  |  |  |  | Bias zero compensation | 5 | FLOAT32 (sample value) | O | - | M | Bias zero compensation. This value is added to the bias value in wistcal calculations. It is intended to compensate for bias zero level offsets. Default 0 |
|  |  |  |  | Ksw port x | x+10 11-60 | FLOAT32 (dB) | O | - | M | Factor for switch port x in linear approximation. |
| Frequency compensation | “FRQC” | 1 | 1 | lambda0 0) | 1 | FLOAT32 (m) | O | - | M | Constant value in linear approximation. Note that this parameter is specified for the wavelength domain.  = k *t + 0 Where t is the time after the pulse start. c The complete compensation also includes look-up tables. See WICA LUTF |
|  |  |  |  | kLambda (k | 2 | FLOAT32 (m/s) | O | - | M | Factor in linear approximation. Note that this parameter is specified for the wavelength domain. |
|  |  |  |  | TimeToStart | 3 | FLOAT64 (s) | O | - | M | Where on the fiber the LUT is valid |
|  |  |  |  | deltaTime | 4 | FLOAT64 (s) | O | - | M | Time step in lut. |
| Start delay | “STRD” | 1 | 1 | Offset | 1 | FLOAT32 (s) | M | - | M | The time to wait after the actuator pulse, before starting to acquire samples. |
| Pulse period | “PULP” | 1 | 1 | ReferencePeriod | 1 | FLOAT64 (s) | O | - | M | The start value to use as time between each actuator pulse. The actual period is optimized by WISTOM to compensate for temperature and resonance. |
|  |  |  |  | T0 | 2 | FLOAT32 (degrees C) | O | - | M | At what temperature the reference period is valid. Period = kT * ( temp - T0) + ref. period |
|  |  |  |  | kT | 3 | FLOAT32 (s/degrees C) | O | - | M | How much the pulse period is changed for each degree. |
|  |  |  |  | EnabTempComp | 4 | UINT8 | O | - | M | This variable is moved to the command PULF COMP. It is left here just for backward compatibility. |
|  |  |  |  | EnabFreqComp | 5 | UINT8 | O | - | M | This variable is moved to the command PULF COMP. It is left here just for backward compatibility. |
| Actuator configuration update time | “ACUT” | 1 | 1 | PulsesToBlock | 1 | UINT16 | M | - | M | Time to hold sampling after a change of actuator parameters. The parameter is given as a number of pulse periods. Value range: 0-63.. WEB2002 only |
| Pulse amplifier attenuation control | “PABC” | 1 | 1 | Attenuation | 1 | FLOAT32 | M | - | M | The pulse D/A-converter output attenuation. The output voltage is multiplied by this factor. Range: 0 - 1. |
| Calibration daemon | “CALD” | 1 | 1 | Enable | 1 | UINT8 | O | - | M | 1 = enabled; 0 = disabled. Not supported in WEB2017 |
| Sampling bias control Works only for R12C and later | “SABC” | 1 | 1 | Bias | 1 | FLOAT32 | O | - | M | WEB2003:The sampling A/D-converter bias correction. The sampled voltage is multipled by this factor. Range: 1 .. 1 WEB2017 R12C and later: The targer value for the ADC min value: Default -3000 |
|  |  |  |  | AutoBiasEnabled | 2 | UINT8 | O | - | M | 1 = enabled; 0 = disabled. |
|  |  |  |  | AutoBiasLevel | 3 | FLOAT32 | O | - | M | WEB2003:The nominal bias level for the auto correction. WEB2017 R12C and laster: ±Window for min level of ADC. Default is 1000 |
| Calibrated spec limits | “FLIM” | 1 | 1 | Enable | 1 | UINT8 | O | - | M | 1 =The use of the limits set in tag 2 and tag 3 are enabled . 0 = The use of the limits set in tag 2 and tag 3 are disabled. |
|  |  |  |  | Start of calib spec | 2 | FLOAT64 (Hz) | O | - | M | Start frequency of calibrated spectrum |
|  |  |  |  | End of calib spec | 3 | FLOAT64 (Hz) | O | - | M | Stop frequency of calibrated spectrum |
| Scan Size | “SCAS” | 1 | 1 | Scan Size | 1 | UINT32 | - | - | M | Always returns 1792 |
| WISTOM filter bandwidth | “FBDW” | 1 | 1 | Filter bandwidth (B) | 1 | FLOAT64 (Hz) | M | - | M | Obsolete, replaced by OPM# FILW (same function). |
| LUT indexing configuration | “CTEL” | 1 | 1 | Start temperature | 1 | FLOAT32 (C) | O | - | M | All LUTs that have a temperature or pulse period dependency use these settings to define the first temperature/pulse period, and how much the temperature/pulses period shall change for each LUT index. Index = (T – startTemp)/tempStep or Index = (P – startPeriod)/periodStep This tag is the start temperature. |
|  |  |  |  | Temperature step | 2 | FLOAT32 (C) | O | - | M | How much the temperature changes for each index in the LUT. |
|  |  |  |  | Start period | 3 | FLOAT32 (seconds) | O | - | M | How much the pulse period changes for each index in the LUT. |
|  |  |  |  | Period step | 4 | FLOAT32 (seconds) | O | - | M | How much the pulse period changes for each index in the LUT. |
|  |  |  |  | Temperature or pulse period | 5 | UINT8 | O | - | M | Either temperature or pulse period is used for LUT indexing. 0 = Use temperature for LUT indexing 1 = Use pulse period for LUT indexing |
| LUT for pulse period. | “LUTP” | 1 | 1 | LUTperiod | 2 | FLOAT32 * 100 | M | - | M | Used in calculation of pulse period  The LUT contains the difference between a real value and the linear equation approximation. The valid range of the LUT is determined by the “CTEL” configuration.  = 0 + kT (T-T0) + LUTP (T) |
| Factor LUT used in calibration. | “LUTF” | 1 | 1 | LUTfactor | 2 | FLOAT32 * 1024 * 100 | M | - | M | Two-dimensional LUT used in the initial power compensation. The expected order of the elements are [0][0], [0][1], …, [0][99], [1][0], [1][1], …, [1][99], …. [1023][99], where the first dimension is the wavelength, with a maximum of 1024 elements, and the second dimension is the temperature dimension with 100 elements. The valid temperature range of the LUT is determined by the “CTEL” configuration and the valid frequency range is determined by the “PWRC” configuration. V(t, T) = LUTF(t, T) {Y(t) + LUT0(T)} |
| Offset LUT used in calibration. | “LUTL” | 1 | 1 | LUTlambda0 | 2 | FLOAT32 * 1024 * 100 | M | - | M | Two-dimensional LUT used in frequency compensation. The LUT contains the difference between a real value and the linear equation approximation. The expected order of the elements are [0][0], [0][1], …, [0][99], [1][0], [1][1], …, [1][99], …. [1023][99], where the first dimension is the wavelength, with a maximum of 1024 elements, and the second dimension is the temperature dimension with 100 elements. The valid temperature range of the LUT is determined by the “CTEL” configuration and the valid frequency range is determined by the “FRQC” configuration. (t, T) = {k + LUTK(T)} t + 0 + LUTL(t, T) |
| Factor LUT used calibration. | “LUTK” | 1 | 1 | LUTklambda | 2 | FLOAT32 * 100 | M | - | M | LUT used in frequency compensation. The LUT contains the difference between a real value and the linear equation approximation. The valid temperature range of the LUT is determined by the “CTEL” configuration. (t,T) = {k + LUTK(T)} t + 0 + LUTL (t,T) |
| Start/stop pulse generation | “SPUG” | 1 | 1 | StartStop | 0x01 | UINT8 | M | - | M | Values: 0 = Stop generation 1 = Start generation |
| Save tables. | “STAB” | 1 | 1 | - | - | - | - | - | - | Save LUT to permanent storage. |
| Calibrated temp limits | “TLIM” | 1 | 1 | Enable | 1 | UINT8 | O | - | M | 1 =The use of the limits set in tag 2 and tag 3 are enabled . 0 = The use of the limits set in tag 2 and tag 3 are disabled. |
|  |  |  |  | Start temp | 2 | FLOAT32 (Celsius ) | O | - | M | Start temperature of calibration |
|  |  |  |  | Stop temp | 3 | FLOAT32 (Celsius) | O | - | M | Stop temperature of calibration |

| ADC-bias on optical channel | “BIAS” | 1 | 1 | Voltage chan A | 1 | FLOAT32 (V) | M | - | M | There are two available ADC channels each coupled to a separate optical frontend slot 0 = Slot A 1 = Slot b |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 1 | Voltage chan B | 2 | FLOAT32(V) |  |  |  |  |

| OpticalADC channel | “CHAN” | 1 | 1 | Channel | 2 | UINT8 | M | - | M | There are two available ADC channels each coupled to a separate optical frontend slot 0 = Slot A 1 = Slot b |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### SMGR:System Management

| Application | App code (ApiApp_t) |
| --- | --- |
| System Parameters | “SMGR” |
| Operation | Op code (ApiOp_t) | Max. User Lvl |  | Data Field | Tag | Data type | Class |  |  | Description |
|  |  | Get | Set |  |  |  | set | get | res |  |
| Restart system | “REST” | - | 2 | RestartType | 1 | UINT16 | M | - | - | Restarts the system. The restart type can be either: 0: Hard reset (all hardware is reset) 1: Soft reset (only software is restarted) |
| IP settings | “IP##” | 3 | 2 | IPAddress | 1 | String | O | - | M | IP address in Ipv4 format (e.g. 192.178.12.4). String with four numbers separated with a dot. |
|  |  |  |  | IPSubnetMask | 2 | String | O | - | M | IP subnet mask in Ipv4 format. (e.g. 255.255.255.0) String with four numbers separated with a dot. |
|  |  |  |  | Gateway | 3 | String | O | - | M | IP address to the default gateway. (e.g. 192.168.1.1) String with four numbers separated with a dot. |
|  |  |  |  | HostName | 4 | String | O | - | M | The host name of this unit. |
|  |  |  |  | MACAddress | 5 | String | - | - | M | Ethernet MAC address string with six hexadecimal numbers separated with a colon. (e. g (00:08:0F:00:01:15) |
|  |  |  |  | TCP/IP listening port | 6 | UINT16 | O | - | M | The port where the WISTOM TCP/IP communication interface listens for new connections. Default value: 7734 |
| Save configuration. | “FLSH” | - | 2 | Side | 1 | UINT8 | M | - | - | 0 = next valid area |
| Serial (RS232) settings | “SER#” | 3 | 2 | Interface | 1 | UINT8 | M | M | M | Which RS232 interface to configure. Must be 1. |
|  |  |  |  | BaudRate | 2 | UINT32 | O | - | M | Baud rate. 1200, 2400, 4800, 9600, 19200, 38400, 57600, 115200. |
|  |  |  |  | DataBits | 3 | UINT8 | O | - | M | Number of data bits. 7 or 8. |
|  |  |  |  | StopBit | 4 | UINT8 | O | - | M | Number of stop bits. 1 or 2. |
|  |  |  |  | Parity | 5 | UINT8 | O | - | M | Parity. None (0), Odd (1) or Even (2). |
| Date and Time | “TIME” | 5 | 3 | Year | 1 | UINT16 | O | - | M | The year as an integer (e.g. 2001). |
|  |  |  |  | Month | 2 | UINT8 | O | - | M | The month as an integer in the range 1-12. |
|  |  |  |  | Day | 3 | UINT8 | O | - | M | The day of the month as an integer in the range 1-31. |
|  |  |  |  | Hour | 4 | UINT8 | O | - | M | The hour of day as an integer in the range 0-23. |
|  |  |  |  | Minute | 5 | UINT8 | O | - | M | The minutes of the hour as an integer in the range 0-59 |
|  |  |  |  | Seconds | 6 | UINT8 | O | - | M | The seconds of the minute as an integer in the range 0-59. |
| Product info | “INFO” | 5 | 1 | HWProductNo. | 1 | String | O | - | O | Hardware product number. Article number. If there are several parts of the same type with different serial numbers or revision, the corresponding data field sequence may be repeated. |
|  |  |  |  | HWIdNo. | 2 | String | - | - | O | Hardware id number. Unique number from temp sensor on WEB. |
|  |  |  |  | HWRevision | 3 | String | - | - | O | Hardware revision. WEB board revision. |
|  |  |  |  | HWSerialNo | 4 | String | O | - | O | Hardware serial number (from temp sensor on web board) |
|  |  |  |  | SensorProductNo | 17 | String | O | - | O | Sensor product number. Article number. |
|  |  |  |  | SensorIdNo. | 18 | String | - | - | O | Sensor id number. Unique number from temp sensor in HOG. |
|  |  |  |  | SensorRevision | 19 | String | O | - | O | Sensor revision. |
|  |  |  |  | SensorSerial | 20 | String | O | - | O | Sensor serial number |
|  |  |  |  | SW product no. | 33 | String | - | - | O | Software product number |
|  |  |  |  | SW revision | 35 | String | - | - | O | Software revision. |
|  |  |  |  | FW revision | 51 | String | - | - | O | Firmware (FPGA software) revision |
|  |  |  |  | Driver revision | 52 | String | - | - | O | WEB2017 driver version |
|  |  |  |  | BootS revision | 53 | String | - | - | O | Linux version |
|  |  |  |  | SwitchS revision | 54 | String | - | - | O | Switch response message.: Reads the ID on SPI channel 0, 1 2 and 3. If no switch is connected, the responce will be 0 |
|  |  |  |  | UnitSerial | 65 | String | O | - | O | The unit serial number for complete unit. |
|  |  |  |  | ProductionDate | 66 | String | O | - | O | Production date of the unit |
|  |  |  |  | Start freq. of calibration | 80 | FLOAT64 (Hz) | - | - | M | Start frequency of calibration |
|  |  |  |  | Stop freq. of calibration | 81 | FLOAT64 (Hz) | - | - | M | Stop frequency of calibration |
|  |  |  |  | Start temp. of calibration | 82 | FLOAT32 (degree) | - | - | M | Start temperature of calibration |
|  |  |  |  | Stop temp. of calibration | 83 | FLOAT32 (degree) | - | - | M | Stop temperature of calibration |
| Hidden commands | “HIDD” | - | 1 |  |  |  |  |  |  | Activate and deactivate shell commands Data 0 = Deactivate , 1 = Activate Currently no function but retained for future needs |
| Temperature | “TEMP” | 4 | 3 | BoardTemp | 1 | FLOAT32 (C) | - | - | M | The electronic board temperature. |
|  |  |  |  | SensorTemp | 2 | FLOAT32 (C) | - | - | M | The sensor temperature. |
|  |  |  |  | Sensor Temp Derivata | 3 | FLOAT32 (C/s) | - | - | M | The sensor temperature derivate, taken as temperature difference over a history of 200 measurements ( ~ 200s) and scaled to (C/s) |
|  |  |  |  | Configured MinTemp | 4 | FLOAT32 (C) | O | - | M | User-configured temperature limits. Only used for temperature alarms (WISTOM will continue to function). Calibration temperature limits are defined in SMGR INFO tags 82 and 83. |
|  |  |  |  | Configured Max Temp | 5 | FLOAT32 (C) | O | - | M |  |
|  |  |  |  | FPGA temp | 6 | FLOAT32 (C) | - | - | M | Returns the temperature reported by the build in temp sensor of the FPGA |
| System running time | UPTI | 5 | - | System Uptime | 1 | FLOAT32 (s) | - | - | M | The time since the unit operating system was started, that is since power on or since last hard restart (SMGR REST 1 0) |
|  |  |  |  | Application uptime | 2 | FLOAT32 (s) |  |  | M | The time since the application started, that is the time since last soft restart (SMGR REST 1 1) |
|  |  |  |  | System load factor filtered over 1 min | 3 | FLOAT32 |  |  | M | The load factor as reported by linux top command. A value below 1 means that the system is operating below maximum load. As it is a two processor system but where only one processor can handle FPGA, the exact maximum is complex. |
| Feature installation | INST | 5 | 1 | SNMP agent | 1 | UINT8 | O | - | M | 0 = Stop and uninstall SNMP agent 1 = Install and start SNMP agent Get |
|  |  |  |  | OWM2 SNMP Agent | 2 | UINT8 | O | - | M | Not supported |
| SNMP configuration | SCFG | 5 | 2 | Agent listening port | 1 | UINT16 | O | - | M | UDP port where the agent listens to incoming requests (agent must be re-started for new setting to take affect.). |
|  |  |  |  | Obsolete | 2 | - | - | - | - | Replaced by SMGR SATR; SMGR SDTR; SMGR SLTR. |
|  |  |  |  | Obsolete | 3 | - | - | - | - | Replaced by SMGR SATR; SMGR SDTR; SMGR SLTR. |
| SNMP Add trap receiver | SATR | - | 2 | Managers trap IP address. | 1 | STRING | M | - | - | The IP address of the manager handling traps (agent must be re-started for setting to take affect.). The maximum number of trap receivers are 30. |
|  |  |  |  | Managers trap listen port | 2 | UINT16 | M | - | - | UDP port where the manager listen to incoming traps (agent must be re-started for new setting to take affect.). |
| SNMP Delete trap receiver | SDTR | - | 2 | Managers trap IP address. | 1 | STRING | M | - | - | The IP address of the manager handling traps (agent must be re-started for setting to take affect.). |
|  |  |  |  | Managers trap listen port | 2 | UINT16 | M | - | - | UDP port where the manager listen to incoming traps (agent must be re-started for new setting to take affect.). |
| SNMP List trap receivers | SLTR | 5 | - | Managers trap IP address. | 1 | STRING | - | - | M | The IP address of the manager handling traps (agent must be re-started for setting to take affect.). Tag 1, IP address, is used to indicate new trap receiver. This means that all data, in any order, after a IP address tag until next IP address tag or end of data belongs to that trap receiver. |
|  |  |  |  | Managers trap listen port | 2 | UINT16 | - | - | M | UDP port where the manager listen to incoming traps (agent must be re-started for new setting to take affect.). |
| LED status | LED# | 5 | - | Power | 1 | UINT8 | - | - | M | Not implemented |
| f |  |  |  |  |  |  |  |  |  |  |

### Login Server

| Application | App code (ApiApp_t) |
| --- | --- |
| Login Server | “LGIN” |
| Operation | Op code (ApiOp_t) | Max. User Lvl |  | Data Field | Tag | Data type | Class |  |  | Description |
|  |  | Get | Set |  |  |  | set | get | res |  |
| Change password | “CPWD” | - | 5 | Password | 2 | String | M | - | - | Sets the password for the currently logged in user. Min 4 and max 20 characters. |
| Change others password | “COPW” | - | 2 | UserName | 1 | String | M | - | - | This operation changes the password of another user. |
|  |  |  |  | Password | 2 | String | M | - | - | The password as a string. Min 4 and max 20 characters. |
| User add | “UADD” | - | 2 | UserName | 1 | String | M | - | - | Null-terminated string with the user name. Min 4 and max 20 characters. |
|  |  |  |  | Password | 2 | String | M | - | - | Null-terminated string with the password Min 4 and max 20 characters. |
|  |  |  |  | UserLevel | 3 | UINT32 | M | - | - | The user level, see section 4. |
|  |  |  |  | Interfaces | 4 | UINT32 | M | - | - | Bitmap over the interfaces where this user is allowed to log in. 0xFFFFFFFF means all interfaces and is reserved for root and admin user. Bit 0: RS232 MMI Bit 1: RS232 other Bit 2: Telnet MMI Bit 3: API over TCP/IP (“socket”) Bit 4: SNMP |
| User delete | “UDEL” | - | 2 | UserName | 1 | String | M | - | - | Null-terminated string with the user name. |
| List user information | “UINF” | 2 | - | UserName | 1 | String | - | M | M | Null-terminated string with the user name. A “*” functions as a wildcard and data fields are repeated for all users that are available in the system. |
|  |  |  |  | UserLevel | 3 | UINT32 |  |  | M | The user level, see section 4. |
|  |  |  |  | Interfaces | 4 | UINT32 |  |  | M | Bitmap over the interfaces where this user is allowed to log in. 0xFFFFFFFF mean all interfaces and is the normal value. Bit 0: RS232 MMI Bit 1: RS232 other Bit 2: Telnet MMI Bit 3: API over TCP/IP (“socket”) Bit 4: SNMP |
| Session info | “SINF” | 4 | - | UserName | 1 | String | - | - | M | Lists all users currently logged in. The tags are repeated for each user. |
|  |  |  |  | ProcessID | 5 | UINT32 | - | - | M | Id number of the API process used by user in tag 1 |

### OCM

| Application | App code (ApiApp_t) |
| --- | --- |
| Optical Channel Monitoring | “OCM#” |
| Operation | Op code (ApiOp_t) | Max. User Lvl |  | Data Field | Tag | Data type | Class |  |  | Description |
|  |  | Get | Set |  |  |  | set | get | res |  |
| OCM evaluation window size | “WINS” | 5 | 2 | Window size | 1 | UINT16 | O | - | M | This parameter defines how many samples that are summed up before judging channel amplitude. Values: 3, 5, 9 or 27 |
| OCM average | “AVRG” | 5 | 2 | Averages | 1 | UINT16 | O | - | M | How many terms that shall be included in a summation of samples (for calculation of mean values). Note that this parameter is multiplied with the number of sub scans inside WISTOM. Range: 2x, x=0-9 |
| OCM enable | “ENAB” | 5 | 3 | Action | 1 | UINT8 | M | - | M | = 0 OCM is disabled. Default = 1 OCM is enabled |

### OPM

| Application | App code (ApiApp_t) |
| --- | --- |
| Optical Performance Monitoring | “OPM#” |
| Operation | Op code (ApiOp_t) | Max. User Lvl |  | Data Field | Tag | Data type | Class |  |  | Description |
|  |  | Get | Set |  |  |  | set | get | res |  |
| OPM average | “AVRG” | 4 | 3 | Averages | 1 | UINT32 | O | - | M | How many consecutive scans that shall be collected and averaged before analysis. Range: 2n, n=0-12 |
| OPM enable | “ENAB” | 5 | 3 | Action | 1 | UINT8 | M | - | M | = 1 OPM is enabled = 0 OPM is disabled |
| OPM power calculation | “CALC” | 3 | 3 | Channel Power | 1 | UINT8 | O | - | M | Calculate channel power with noise or without noise. 0 = channel power is calculated without noise. 1= channel power is calculated with noise. Default |
| OPM power calculation | “CNFG” | 1 | 1 | Envelope average samples | 1 | UINT8 | O | - | M | Number of samples used for averaging the noise envelope. Range: 1 - 255. Default: 12 |
|  |  |  |  | Bias moving average. | 2 | UINT16 | O | - | M | Number of extra terms that SW should average Bias value used by OPM. Range: 1-511, Nominal value 1. |
|  |  |  |  | Spectrum moving average | 3 | UINT16 | O | - | M | Number of extra terms that SW should average spectrum samples. Value used by OPM OSNR calculation. Range: 1-31, Nominal value 1. |
|  |  |  |  | Side attenuation | 4 | UINT8 | O | - | M | 0 = don’t calculate side attenuation. 1 = calculate side attenuation. Default |
| OPM OSNR configuration | “OSNR” | 4 | 2 | OsnrVariant | 1 | UINT8 | O | - | M | OSNR calculated accordingly =1: osnr =10*lg(channelpower/noisepower) + 10*lg(measuredbandwith/ReferenceFilterBandwith) =2: osnr = 10*lg(channelpower/noisepower) + 10*lg(OpticalFiltrBandwith/ReferenceFilterBandwith) =3: osnr = 10*lg(channelpower/noisepower) Default: 1 |
|  |  |  |  | Measured bandwidth | 2 | FLOAT64 (nm) | O | - | M | The measured bandwidth of the optical filter in the Wistom unit. Used for calculation of OSNR Range: 1e-12 to 0.3 decimal. Nominal value: 0.08 |
|  |  |  |  | Reference bandwidth | 3 | FLOAT64 (nm) | O | - | M | The reference bandwidth of the optical filter in the Wistom unit. Used for calculation of OSNR Range: 1e-12 to 0.3 decimal. Nominal value: 0.10 |
|  |  |  |  | Max distance for OSNR noise measurement | 4 | FLOAT64 (Hz) | O | - | M | This value specifies a distance in Hz. The value is used as the highest allowed distance from a channel’s center frequency to the point where the noise level used in OSNR is measured. Range: 1e9 to 5.5e12 decimal. |
|  |  |  |  | Low limit in noise estimation | 5 | FLOAT64 (W/Hz) | O | - | M | This value specifies a minimum value. The value is used as the lowest allowed value of the spectral density of the noise estimation in OSNR calculation Range: 1e-25 to 1e-10 decimal. |
|  |  |  |  | Noise level samples | 6 | UINT32 | O | - | M | The number of samples that are used for averaging the noise level. Used for calculation of OSNR Range: 1 – 100 decimal. Nominal value: 10 |
| OPM channel configuration | “CHCO” | 3 | 3 | Process configured channels | 1 | UINT8 | O | - | M | 0 = process all channels. I.e. looks through entire spectrum and reports all found channels. 1 = process only the channels configured in spectrum parameters. See SPEC CHNL. |
|  |  |  |  | Static integration interval | 2 | UINT8 | O | - | M | 0 = dynamic integration interval (default) 1 = use static integration interval according to tags 1 and 3. If tag 1 is 0: use static integration interval set in tag 3. The interval is the same for all peaks. If tag 1 is 1: use static integration interval set in SPEC CHNL tag 26. The integration interval is unique for each channel. |
|  |  |  |  | Peak integration interval | 3 | FLOAT64 (Hz) | O | - | M | This value specifies a distance in Hz. The value is used as a distance on both sides of the channel’s center frequency to the points where begin and where to stop integration when calculating the area under the channel peak. Range: 1e9 to 200e9 decimal. |
|  |  |  |  | Search for unconfigured channels | 4 | UINT8 | O | - | M | This tag is only valid when OPM is in configured mode (see tag 1) 0 = Only configured channels are searched for (default) 1 = Search for channels besides the configured. Any new channels will generate the alarm “OPM unconfigured channel found” (see section 5.3.2). |
|  |  |  |  | Channel auto-numbering base | 5 | UINT16 (GHz) | O | - | M | In automatic mode (see tag 1), channels are numbered according to their frequency, aligned with the grid given in this parameter. Range: 10 – 1000, default is 50 GHz. E.g. channels with frequency 194.1, 194.15, 194.2, 194.25 THz are numbered 19410, 19420, 19420, and 19430 respectively when this tag is 100. If the tag is set to 50, the same channels are numbered 19410, 19415, 19420, and 19425. When the tag is set to for example 25, the generated channel number does not fit within an UINT16, so the most significant figures are truncated; 4100, 4150, 4200, 4250. The frequency 194.125 THz will in this case generate the channel number 4125. Note that channels may receive the same channel number if they are located within the frequency grid. |
| OPM frequency options | FRQO | 5 | 3 | Frequency calc mode | 1 | UINT8 | M | - | M | =0 Frequency is reported as the interpolated peak value =1 Frequency is reported as the center frequency at the FWHM level. |
| OPM output spectrum | “OUTP” | 1 | 1 | Signal data | 1 | UINT8 | O | - | M | Configure the way TSPC, FSPC, WSPC represents its amplitude spectrum data. 0 = don’t print signals data spectrum. 1 = print signals data spectrum. Default The signal is added to the noise spectrum if so specified in tag 2. If both tag1 and tag2 is set to zero, noise added to signal is presented. |
|  |  |  |  | Noise data | 2 | UINT8 | O | - | M | 0 = don’t print signals noise spectrum. 1 = print signals noise spectrum. Default This signal is added to the signal spectrum if so specified in tag 1. |
| OPM raw data | “RAWD” | 1 | - | Spectrum | 1 | UINT32 (length) Length * FLOAT32 | - | - | M | Measured samples normalized to the number of averages. |
|  |  |  |  | SwitchPort | 100 | UINT8 | - | O | O | The switch port that the spectrum shall be fetched from. If this tag is omitted in the get request, the value in OPM# SWHA is used (for backward compatibility). This tag is only sent in the get result if the number of inputs is more than one (1). |
| OPM time spectrum | “TSPC” | 1 | - | Time table | 2 | UINT32 (length) length * FLOAT32 (s) | - | - | M | Measured data. This is the time of measurement for each sample. |
|  |  |  |  | Power table | 4 | UINT32 (length) length * FLOAT32 (W/Hz) | - | - | M | Measured data. This is the measured power at each time in the timetable. |
|  |  |  |  | SwitchPort | 100 | UINT8 | - | O | O | The switch port that the spectrum shall be fetched from. If this tag is omitted in the get request, the value in OPM# SWHA is used (for backward compatibility). This tag is only sent in the get result if the number of inputs is more than one (1). |
| OPM frequency spectrum | “FSPC” | 4 | - | Frequency table | 3 | UINT32 (length) length * FLOAT32 (Hz) | - | - | M | Measured data. This is the frequency of each measured value. |
|  |  |  |  | Power table | 4 | UINT32 (length) length * FLOAT32 (W/Hz) | - | - | M | Measured data. This is the measured power at each frequency in the frequency table. |
|  |  |  |  | Switch Port | 100 | UINT8 | - | O | O | The switch port that the spectrum shall be fetched from. If this tag is omitted in the get request, the value in OPM# SWHA is used (for backward compatibility). This tag is only sent in the get result if the number of inputs is more than one (1). |
| OPM wavelength spectrum | “WSPC” | 4 | - | Wavelength table | 5 | UINT32 (length) length * FLOAT32 (m) | - | - | M | Measured data. This is the wavelength of each measured value. |
|  |  |  |  | Power table | 4 | UINT32 (length) length * FLOAT32 (W/m) | - | - | M | Measured data. This is the measured power at each frequency in the frequency table. |
|  |  |  |  | SwitchPort | 100 | UINT8 | - | O | O | The switch port that the spectrum shall be fetched from. If this tag is omitted in the get request, the value in OPM# SWHA is used (for backward compatibility). This tag is only sent in the get result if the number of inputs is more than one (1). |
| OPM compact spectrum | “CSPC” | 4 | - | Interleave | 101 | UINT8 | - | O | M | This command is used to preserve network bandwidth when viewing spectrums. The accuracy will be somewhat lesser than for the full spectrums. The interleave is used to fetch every 2, 4, 8, … sample in the spectrum. Increase the index with one for every fetched spectrum to get the complete image in a number of fetches. Default: 1 |
|  |  |  |  | Interleave index | 102 | UINT8 | - | O | M | Start with this sample and then fetch samples according to the “interleave”. Default: 0 |
|  |  |  |  | Include tables | 103 | UINT8 | - | O | - | It is optional to fetch the power table, the wavelength table, or both. 1: Include wavelength table 2: Include power table 3: Include both tables Default: 4 |
|  |  |  |  | Power table | 40 | UINT32 (length) length * UINT16 ((dBm+10) *-100) | - | - | O | A compact version of the power table. The samples are given as integers denoting 100th of a dBm counting from -10; e.g. -34.231 dBm is sent as the integer 2423. Thus, the power range is -10 to -75.535 dBm. This tag is only included if set by the tag “include tables” above. |
|  |  |  |  | Wavelength table | 50 | UINT32 (length) length * UINT16 ((nm-start) *1000 | - | - | O | A compact version of the wavelength table. The sample positions are given as integers denoting the offset from the “wavelength table start” in picometers. Thus, the maximum wavelength range is 65.535 nanometers. Since the “start wavelength” is the highest one, the offsets are negative. I.e. wvl = start – offset. This tag is only included if set by the tag “include tables” above. |
|  |  |  |  | Wavelength table start | 51 | UINT32 | - | - | O | Start value for the wavelength table. The wavelength 1525.923 nm is given as the integer 1525923. This tag is only included if set by the tag “include tables” above. |
|  |  |  |  | SwitchPort | 100 | UINT8 | - | O | O | The switch port that the spectrum shall be fetched from. If this tag is omitted in the get request, the value in OPM# SWHA is used (for backward compatibility). This tag is only sent in the get result if the number of inputs is more than one (1). |
| OPM channel status | “CHNL” | 5 | - | ChannelId | 1 | UINT16 | - | M | M | This value specifies one specific channel. In OPM automatic mode, the channel number is based on the frequency according to OPM# CHCO tag 5, in configured mode the channel number is based on the configured number. |
|  |  |  |  | SwitchPort | 100 | UINT8 | - | O | O | The switch port that this channel emanates from. This tag is mandatory in the get request when OPM is in Automatic mode. This tag is only sent in the result if the number of inputs is more than one (1). |
|  |  |  |  | FreqC | 2 | FLOAT64 (Hz) | - | - | M | Central frequency |
|  |  |  |  |  | 3 |  | - | - | - | Obsolete. See OPM# FRQO |
|  |  |  |  | FWHM | 4 | FLOAT64 (Hz) | - | - | M | FWHM |
|  |  |  |  | Amplitude | 5 | FLOAT64 (W/Hz) | - | - | M | Amplitude at the central frequency of the channel. |
|  |  |  |  | PowerC | 6 | FLOAT64 (W) | - | - | M | Central power |
|  |  |  |  | OSNR | 7 | FLOAT64 (dB) | - | - | M | Optical signal to noise ratio of the channel |
|  |  |  |  | Channel spacing | 8 | FLOAT64 (Hz) | - | - | M | Channel spacing used for measurement of OSNR. This is the distance from the peak to the noise measurement position, i.e. half the channel spacing. |
|  |  |  |  | Status PowerC | 9 | UINT8 | - | - | O | Status of measured central power related to configured nominal value. OK 0x01 Not found 0x04 Warning Low 0x10 Warning High 0x20 Alarm Low 0x40 Alarm High 0x80 NOTE! This tag is only reported if the channel is properly configured and channel processing is set to analyze configured only. |
|  |  |  |  | Status FreqC | 10 | UINT8 | - | - | O | Status of measured central frequency related to configured nominal value. OK 0x01 Not found 0x04 Warning Low 0x10 Warning High 0x20 Alarm Low 0x40 Alarm High 0x80 NOTE! This tag is only reported if the channel is properly configured and channel processing is set to analyze configured only. |
|  |  |  |  | Status OSNR | 11 | UINT8 | - | - | O | Status of measured OSNR. OK 0x01 Not found 0x04 Warning 0x10 Alarm 0x40 NOTE! This tag is only reported if the channel is properly configured and channel processing is set to analyze configured only. |
|  |  |  |  | Delta power | 12 | FLOAT64 (dB) | - | - | O | The difference actual power – configured power NOTE! This tag is only reported if the channel is properly configured and channel processing is set to analyze configured only. |
|  |  |  |  | Delta frequency | 13 | FLOAT64 (Hz) | - | - | O | The difference actual frequency – configured frequency NOTE! This tag is only reported if the channel is properly configured and channel processing is set to analyze configured only. |
|  |  |  |  | OSNR margin | 14 | FLOAT64 (dB) | - | - | O | The difference actual OSNR – OSNR alarm level NOTE! This tag is only reported if the channel is properly configured and channel processing is set to analyze configured only. |
|  |  |  |  | Time stamp | 29 | FLOAT32 (s) | - | - | M | Time stamp giving number of seconds since unit start. The resolution is approximately 1 millisecond. |
| OPM all channel status | “CHAL” | 5 | - | This operation returns all calibration data in one consecutive sequence. The data information is exactly the same as for OPM# CHNL, except that the tags are repeated for all channels. Tag 1, ChannelId, is used to indicate new channel. This means that all data, in any order, after a ChannelId tag until next ChannelId tag or end of data belongs to that channel. |  |  |  |  |  |  |
| OPM calibration data | “CALD” | 1 | - | ChannelId | 1 | UINT16 | - | M | M | This value specifies one specific channel. In OPM automatic mode, the channel number is based on the frequency according to OPM# CHCO tag 5, in configured mode the channel number is based on the configured number. |
|  |  |  |  | SwitchPort | 100 | UINT8 | - | O | O | The switch port that this channel emanates from. This tag is mandatory in the get request when OPM is in Automatic mode. This tag is only sent in the result if the number of inputs is more than one (1). |
|  |  |  |  | FreqC | 2 | FLOAT64 (Hz) | - | - | M | Central frequency |
|  |  |  |  | FreqFWHM | 3 | FLOAT64 (Hz) | - | - | M | Frequency based on FWHM |
|  |  |  |  | FWHM | 4 | FLOAT64 (Hz) | - | - | M | FWHM |
|  |  |  |  | Amplitude | 5 | FLOAT64 (W/Hz) | - | - | M | Amplitude at the central frequency of the channel. |
|  |  |  |  | PowerC | 6 | FLOAT64 (W) | - | - | M | Central power |
|  |  |  |  | OSNR | 7 | FLOAT64 (dB) | - | - | M | Optical signal to noise ratio of the channel |
|  |  |  |  | Channel spacing | 8 | FLOAT64 (Hz) | - | - | M | Channel spacing used for measurement of OSNR |
|  |  |  |  | Status PowerC | 9 | UINT8 | - | - | O | Status of measured central power related to configured nominal value. OK 01 Not found 04 Warning Low 10 Warning High 20 Alarm Low 40 Alarm High 80 NOTE! This tag is only reported if the channel is properly configured and channel processing is set to analyze configured only. |
|  |  |  |  | Status FreqC | 10 | UINT8 | - | - | O | Status of measured central frequency related to configured nominal value. OK 01 Not found 04 Warning Low 10 Warning High 20 Alarm Low 40 Alarm High 80 NOTE! This tag is only reported if the channel is properly configured and channel processing is set to analyze configured only. |
|  |  |  |  | Status OSNR | 11 | UINT8 | - | - | O | Status of measured OSNR. OK 01 Not found 04 Warning 10 Alarm 40 NOTE! This tag is only reported if the channel is properly configured and channel processing is set to analyze configured only. |
|  |  |  |  | Time of peak | 12 | FLOAT64 (s) | - | - | M | Time from beginning of scan to peak maximum. This is a special function used during calibration. |
|  |  |  |  | Sample number | 13 | UINT32 | - | - | M | Sample number of peak. |
|  |  |  |  | Bias | 14 | FLOAT32 | - | - | M | Bias level for spectrum. |
|  |  |  |  | Raw data | 15 | FLOAT64 | - | - | M | Raw data for peak |
|  |  |  |  | Raw data from FPGA | 16 | UINT32 | - | - | M | Raw data for peak read from the FPGA. |
|  |  |  |  | Attenuation L1 | 17 | FLOAT64 | - | - | M | Relation between spectral density at -12.5 Ghz from center freq and spectral density at center freq. |
|  |  |  |  | Attenuation R1 | 18 | FLOAT64 | - | - | M | Relation between spectral density at +12.5 Ghz from center freq and spectral density at center freq. |
|  |  |  |  | Attenuation L2 | 19 | FLOAT64 | - | - | M | Relation between spectral density at -25 Ghz from center freq and spectral density at center freq. |
|  |  |  |  | Attenuation R2 | 20 | FLOAT64 | - | - | M | Relation between spectral density at +25 Ghz from center freq and spectral density at center freq. |
|  |  |  |  | Temperature | 21 | FLOAT32 (Celsius) |  |  | M | Temperature for this spectrum |
|  |  |  |  | Time Between Samples | 22 | FLOAT32 (s) |  |  | M | Time between samples for this spectrum |
|  |  |  |  | Peak integration start point | 23 | FLOAT64 (Hz) |  |  | M | Peak integration algorithms start frequency (foot) |
|  |  |  |  | Peak integration end point | 24 | FLOAT64 (Hz) |  |  | M | Peak integration algorithms end frequency (end) |
|  |  |  |  | Peak noise power | 25 | FLOAT64 (W) |  |  | M | Integrated power of noise. Integration between start and end of peak. |
|  |  |  |  | OSNR low frequency | 26 | FLOAT64 (Hz) |  |  | M | Low frequency point of OSNR measurement. |
|  |  |  |  | OSNR high frequency | 27 | FLOAT64 (Hz) |  |  | M | High frequency point of OSNR measurement. |
|  |  |  |  | Pulse period | 28 | FLOAT32 (s) |  |  | M | Pulse period averaged during the data acquisition. |
|  |  |  |  | Time stamp | 29 | FLOAT32 (s) |  |  | M | Time stamp giving number of seconds since unit start. |
| OPM calibration data for all channels | “CALA” | 1 | - | This operation returns all calibration data in one consecutive sequence. The data information is exactly the same as for OPM# CALD, except that the tags are repeated for all channels. Tag 1, ChannelId, is used to indicate new channel. This means that all data, in any order, after a ChannelId tag until next ChannelId tag or end of data belongs to that channel. |  |  |  |  |  |  |
| OPM threshold | “TRSH” | 5 | 3 | TrshValue | 1 | FLOAT64 (W/Hz) | O | - | M | This value specifies a threshold value. When a peak is discovered and it’s max amplitude is found the peak will be accepted if the max amplitude value minus noise level at the point for the max amplitude is greater than the threshold value. Low limit: 1e-23 |
|  |  |  |  | TRSH value in W | 2 | FLOAT64 (W) | O | - | M | This value specifies a threshold value. Same as above but in unit Watt. When a peak is discovered and it’s max amplitude is found the peak will be accepted if the max amplitude value minus noise level at the point for the max amplitude is greater than the threshold value. This value is tag 1 times the value of the OPM# FILW operation. Low limit: 1e-13 |
|  |  |  |  | Maximum number of peak candidates | 3 | UINT16 | O | - | M | Maximal number of peak candidates that is to be selected for test against the peak criteria values in OPM# PCRI operation. Range: 1 to 1024 Default: 512 |
| OPM minlevel | “MINL” | 5 | 3 | Min level | 1 | FLOAT64 (W/Hz) | O | - | M | This value specifies a minimum value. The value is used as the lowest allowed value of the spectral density in the printed spectrum. Low Limit: 1.0e-28 |
|  |  |  |  | Min level in W | 2 | FLOAT64 (W) | O | - | M | This value specifies a minimum value. Same as above but in unit Watt. This value specifies a minimum value. The value is used as the lowest allowed value of the spectral density in the printed spectrum. This value is tag 1 times the value the OPM# FILW operation. Low Limit: 1.0e-18 |
| OPM peak criteria | “PCRI” | 3 | 3 | Foot and End Criteria | 1 | FLOAT64 | O | - | M | This value specifies a start and end level for a peak. The peak amplitude is multiplied with this value and the result will represent the start and end criteria. 0.001< Range <1 |
|  |  |  |  | ClosestPeakCri | 2 | FLOAT64 (Hz) | O | - | M | If several peaks are detected within this interval only the one with highest amplitude will be accepted. 1.0e+9 < Range < 1.0e+14 |
|  |  |  |  | MinPowerCri | 4 | FLOAT64 (W) | O | - | M | If a peak will be accepted the channel power has to be higher than this value. Low Limit: 1.0e-12 |
|  |  |  |  | EnvWindow | 5 | FLOAT64 (Hz) | O | - | M | The minimum distance between neighboring channels. This parameters tells WISTOM how to separate wide peaks from the noise floor (i.e. it controls how the noise floor envelope is calculated) Typical value is the minimum channel distance. Default. 12.5 Ghz 1.0e+9 < Range < 1.0e+14 |
|  |  |  |  | Persistence mode | 6 | UINT8 | O | - | M | Enable/Disable persistence mode. |
| OPM total power | “TPWR” | 4 | 3 | Switch port | 100 | UINT8 | - | O | O | The switch port that the calculation is performed on. If this tag is omitted in the get request, the value in OPM# SWHA is used (for backward compatibility). This tag is only sent in the get result if the number of inputs is more than one (1). |
|  |  |  |  | Start interval | 1 | FLOAT64 (Hz) | O | - | M | Start of integration interval. Range: see SMGR INFO tag 80. |
|  |  |  |  | End interval | 2 | FLOAT64 (Hz) | O | - | M | End of integration interval. Range: see SMGR INFO tag 81. |
|  |  |  |  | Power | 3 | FLOAT64 (W) | - | - | M | Total spectrum power over specified interval. Only valid if Start interval is less than End interval and within calibrated range. If not, the value 0 W is returned. |
| OPM filter width | “FILW” | 4 | 3 | Filter width (B) | 1 | FLOAT64 (Hz) | M | - | M | This parameter is used to convert the samples in a spectrum to estimated power. I.e. by multiplying with this factor, a peak value in W/Hz is converted to W. Note that this parameter is not used for any calculations, only for the spectrum display. |
| OPM system filter bandwidth | SFBW | 1 | 1 | Use as peak width criteria. | 1 | UINT8 | O | - | M | =1 Use as peak minimum width criteria. =0 Do not use as peak minimum width criteria. Default 1. |
| OPM switch handling | SWHA | 4 | 4 | Spectrum port | 100 | UINT8 | O | - | M | The switch port that fetched spectrums shall emanate from. Value 1-16 (if the corresponding switch port is installed, see SPEC SWIN). |

### Pulse frequency

| Application | App code (ApiApp_t) |
| --- | --- |
| Pulse frequency control | “PULF” |
| Operation | Op code (ApiOp_t) | Max. User Lvl |  | Data Field | Tag | Data type | Class |  |  | Description |
|  |  | Get | Set |  |  |  | set | get | res |  |
| Compensation enable | “COMP” | 1 | 1 | Temperature compensation | 1 | UINT8 | O | - | M | Temperature compensation = 0 is disabled = 1 is enabled |
|  |  |  |  | Frequency compensation | 2 | UINT8 | O | - | M | Frequency compensation = 0 is disabled = 1 is enabled |
|  |  |  |  | Temperature sensor select | 3 | UINT8 | O | - | M | = 1 Use WEB temperature sensor = 2 Use HOG temperature sensor |
| Pulse resonance spectrum | “PULS” | 1 | - | Pulse spectrum | 1 | UINT32 (length) length * FLOAT32 | - | - | M | The sampled value at the pulse resonance A/D-converter (measured pulse). The data length is 2048 samples with a time step of 10 ns. |
| Frequency Z regulator | “REGZ” | 1 | 1 | X Reference pos | 1 | FLOAT32 | O | - | M | Reference point in Time sample axis for Zero detection pulse regulation. Multiply with number of subscans to get cross point. Typical: 32 (sample) |
|  |  |  |  | Y Reference pos | 2 | FLOAT32 | O | - | M | Reference point for amplitude sample axis Zero detection pulse regulation. Typical: 128 (sample) |
|  |  |  |  | Number of Z samples | 3 | UINT8 | O | - | M | Number of samples used in second order curve approximation for Zero cross point detection. Divide with 8 and multiply with number of subscans to get actual number used. Internally limited to more than 2. Range: 3 -24 Typical: 24 |
| Frequency regulator values | “REGV” | 1 | - | Reg input | 1 | FLOAT64 | - | - | M | Q value diff between the two last read values |
|  |  |  |  | Reg output | 2 | FLOAT64 | - | - | M | PID regulator output ??? |
|  |  |  |  | Q zero | 3 | FLOAT64 | - | - | M | Offset from zero crossing point. |
|  |  |  |  | Q TopToTop | 4 | FLOAT64 | - | - | M | The value between the largest and lowest value in the RPS spectrum |
|  |  |  |  | Q zero at max | 5 | FLOAT64 | - | - | M | Offset from Zero cross at best frequency when running interval regulation. |
|  |  |  |  | Current freq | 6 | FLOAT64 | - | - | M | Current tested resonance frequency. |
|  |  |  |  | Last written freq | 7 | FLOAT64 | - | - | M | Input target value |
|  |  |  |  | Q square sum | 8 | FLOAT64 | - | - | M | Obsolete ! Q value. |
|  |  |  |  | Best freq | 9 | FLOAT64 | - | - | M | The current best resonance frequency.( Highest Q value) calculated by RPS. |
|  |  |  |  | Temp freq | 10 | FLOAT64 | - | - | M | Calculated resonance frequency from measured temperature. |
|  |  |  |  | Sensor mean | 11 | FLOAT64 | - | - | M | Mean value of amplitude samples |
| Frequency regulator control | “REGC” | 1 | - | Update timer | 1 | FLOAT32 | - | - | M | Temperature update timer (seconds) Typical: 1 |
|  |  |  |  | Regulator step | 2 | FLOAT64 | - | - | M | Frequency scanning regulator step (frequency in [s]) Typical: 0.5e-10 |
|  |  |  |  | Regulator Interval | 3 | FLOAT64 | - | - | M | Frequency scanning regulator Interval (frequency in [s]) Typical: 7e-11 |
|  |  |  |  | Q mode | 4 | UINT8 | - | - | M | Not used 0 = Use Zero cross point detection. // 1 = Use maximum peek-to-peek RPS value Default: 1 (for new units but 0 for released units) |
|  |  |  |  | Regulator mode | 5 | UINT8 | - | - | M | Not used Regulator type for frequency scanning 1 = Use interval frequency scanning 2 = Use Zero cross detection frequency Default: 1 (for new units but 2 for released units) |
|  |  |  |  | Frequency diff window | 6 | FLOAT32 | - | - | M | Maximum difference between calculated temp frequency and frequency scanning value. [s] Value 0 turns off function. Typical: 0 ( off ) |
|  |  |  |  | Points in least square method | 7 | UINT8 | - | - | M | Points in least square method for calculation of top detection Q values. Divide with 8 and multiply with number of subscans to get actual number used. Internally limited to more than 2. Typical: 24 |
|  |  |  |  | Polynomial order | 8 | UINT8 | - | - | M | Polynomial order in least square method. Typical: 3 |
|  |  |  |  | Temperature derivate factor | 9 | UINT32 | - | - | M | Factor to adjust regulator and sampling time according to temperature derivate. Value “RESC” tag 2 is overridden. Value 0 means that no temp derivate adjustment is done, “RESC” tag 2 is used. Range: 0 -12 Typical: 0 |
| Pulse resonance configuration | “RESC” | 1 | 1 | Measurement Start Delay | 1 | UINT16 | M | - | M | The time to wait after issuing the pulse before starting to sample at the other end (the pulse resonance measurement A/D-converter), value in samples from pulse issuing. Typical: 2005 |
|  |  |  |  | Averaging | 2 | UINT32 | M | - | M | The number of scans to add up samples (received from the pulse measurement A/D-converter) before releasing the result. Range: 2x, where x= 5-15 decimal. The averaging max value is dependent of the no. of sub scans used by the system. Max internal value is 65536 => max value 8192 @ 8 sub scans. Typical: 1024 |
|  |  |  |  | Unstable averaging | 3 | UINT32 | M | - | M | NOT USED: Same as tag 2 with the difference that this value is used when pulse frequency control is not in a stable state (e.g. system start-up). Typical: 32 |
| Repeat time event ctrl | “RTEC” | 1 | 1 | Threshold (s) | 1 | FLOAT64 | M | - | M | Threshold used by the event controller to determine repeat time have changed enough to notify subscribers. Typical: 1 e-9 |
| Frequency PID regulator values | “REGP” | 1 | - | P | 1 | FLOAT64 | O | - | M | Duplicate of PULF REGC tag 1 |
|  |  |  |  | P | 2 | FLOAT64 | O | - | M | Proportional regulator value. Used in WEB2017 |
|  |  |  |  | I | 3 | FLOAT64 | O | - | M | NOT USED:Proportional regulator integration value, currently not used |
|  |  |  |  | D | 4 | FLOAT64 | O | - | M | NOT USED:Proportional regulator differential value, currently not used |
|  |  |  |  | Maxout | 5 | FLOAT64 | O | - | M | NOT USED:controllerData.MaxOut |
|  |  |  |  | Maxint | 6 | FLOAT64 | O | - | M | NOT USED:controllerData.MaxInt |
|  |  |  |  | target | 7 | FLOAT64 | O | - | M | NOT USED:controllerData.Target |
|  |  |  |  | Last written freq | 8 | FLOAT64 | O | - | M | Duplicate from pulf regc |
|  |  |  |  | Q square sum | 9 | FLOAT64 | O | - | M | Duplicate from pulf regc |
|  |  |  |  |  | 10 | UINT8 | O | - | M | Duplicate from pulf regc |
|  |  |  |  |  | 11 | UINT8 | O | - | M | Duplicate from pulf regc |
|  |  |  |  |  | 12 | UINT8 | O | - | M | Duplicate from pulf regc |

### setAlarm Handler

| Application | App code (ApiApp_t) |
| --- | --- |
| Alarm Handler | “ALMH” |
| Operation | Op code (ApiOp_t) | Max. User Lvl |  | Data Field | Tag | Data type | Class |  |  | Description |
|  |  | Get | Set |  |  |  | set | get | res |  |
| Alarm status | ALRM | 5 | - | Alarm id | 1 | UINT16 | - | O | M | Alarm id, see section 5.3 for valid id |
|  |  |  |  | Alarm sub id | 2 | UINT16 | - | O | M | Alarm sub id, see section 5.3 for valid sub id |
|  |  |  |  | Alarm status | 3 | UINT32 | - | O | M | Status of alarm, see section 5.3 for valid status. |
|  |  |  |  | Uptime in millisecond ticks | 4 | UINT32 | - | - | O | Uptime in millisecond ticks at time of alarm. Wraps after 49 days. This tag is only included if ALMH TITY tag 1 is 3. |
|  |  |  |  | Uptime in 10 msec ticks | 5 | UINT32 | - | - | O | SNMP style. Uptime in 10 msec ticks at time of alarm. Wraps after 497 days. This tag is only included if ALMH TITY tag 1 is 1. |
|  |  |  |  | Seconds since 1970-01-01 00:00 | 6 | UINT32 | - | - | O | Unix style. Seconds since 1970-01-01 00:00 at time of alarm. Wraps year 2106. This tag is only included if ALMH TITY tag 1 is 2 or 4. |
|  |  |  |  | Millisecond fraction for Unix style time | 7 | UINT16 | - | - | O | The millisecond fraction for the Unix style time stamp reported in tag 5. This tag is only included if ALMH TITY tag 1 is 4. |
| Alarm subscribe | SUBS | 4 | 4 | Alarm id | 1 | UINT16 | M | - | M | Subscribe to alarm with id. |
|  |  |  |  | Alarm sub id | 2 | UINT16ge | O | - | M | Alarm sub id to subscribe to. If omitted default is all. |
|  |  |  |  | Alarm status | 3 | UIN32 | O | - | M | Alarm status to subscribe to. If omitted default is all. |
|  |  |  |  | Subscribe mode | 5 | UINT8 | M | - | M | Defines if all alarms or only the new alarms should be subscribed to. 0 = All alarms, 1 = Only new alarms |
|  |  |  |  | Token | 6 | UINT16 | M | - | M | Returned in alarm message header as a user identifier. Also used as identifier when unsubscribing. |
| Alarm unsubscribe | UNSU | - | 4 | Token | 6 | UINT16 | M | - | - | Unsubscribe to all alarms with token. It is only possible to unsubscribe to alarms associated with connection from where request is issued. |
| Alarm time stamp type | TITY | 4 | 3 | Time stamp type | 1 | UINT8 | M | - | M | Alarm timestamp type: =0 No time stamp =1 SNMP style. Uptime in 10 msec ticks. Wraps after 497 days. (default) =2 Unix style. Seconds since 1970-01-01 00:00. Wraps year 2106. =3 Uptime in millisecond ticks. Wraps after 49 days. =4 Unix style with milliseconds. UINT32 with seconds and an UINT16 for the millisecond fraction.. NOTE. This also affects what the command identifier for alarms will be. See section 3.2.6. |

### Wistsense

The Wistsense add functionality for using Wistom as a sensor interrogator.  To work, the Wistom unit needs to be build as a WistSense with an ASE lightsource, wavelength references and a circulator.

| Application | App code (ApiApp_t) |
| --- | --- |
| Wistsens | “WSNS” |
| Operation | Op code (ApiOp_t) | Max. User Lvl |  | Data Field | Tag | Data type | Class |  |  | Description |
|  |  | Get | Set |  |  |  | set | get | res |  |
| Next peak frequencies | “NEXT” | 5 | N/A | Peak frequencies | 101-116 | FLOAT64 | - | - | O | Peak frequencies for all sensor ports (Hz). Possible ports range from 1-16. |
|  |  |  |  | Peak widths | 201-216 | FLOAT64 | - | - | O | Peak widths for all sensor ports (Hz). |
|  |  |  |  | Peak amplitudes | 221-236 | FLOAT64 | - | - | O | Peak amplitudes for all sensor ports. |
|  |  |  |  | Interferometer linear fit | 3 | FLOAT64 | - | - | M | Coefficients and R-value for the linear fit of samples to zero-crossings for the interferometer difference spectrum. In order: slope, intercept, r-value |
|  |  |  |  | Absolute reference linear fit | 4 | FLOAT64 | - | - | M | Coefficients and R-value for the linear fit of measured absorption lines to tabulated frequency values. In order: slope, intercept, r-value |
|  |  |  |  | Number of data points. | 5 | UINT32 | - | - | M | In order: number of reference lines (# data points in reference linear fit), number of zero-crossings (# points in interferometer linear fit) |
|  |  |  |  | First and last crossing | 6 | FLOAT64 | - | - | M | First and last crossing in diffspectrum used for calibration. |
| Operation | Op code (ApiOp_t) | Max. User Lvl |  | Data Field | Tag | Data type | Class |  |  | Description |
|  |  | Get | Set |  |  |  | set | get | res |  |
| Spectrum data | “DATA” | 5 | N/A | Next sensor channel | 1 | U8 | M |  |  | Normalized spectrum from sensor port. |
|  |  |  |  | Normalized spectrum | 2 | U8 | O |  |  | 0: Not normalized. 1: Normalized with white light spectrum |
|  |  |  |  | Spectrum type | 10 | U8 |  | O |  | Select which data should be returned: 1: Sensor spectrum (port specified by earlier set WSNS Spec 1 port) 2: Wavelength reference spectrum, 3: Interferometer spectrum, 4: White light spectrum 5: Frequency scale (Hz) Array of FLOAT 32 |
|  |  |  |  |  | 11 | UINT32 (length) Length * FLOAT32 |  |  | M | Spectrum or data as returned as selected by tag 10 |
| Parameters | “PARA” | 5 | 5 | Peak height for sensor peaks. | 1 | FLOAT64 | O | - | M | The minimum vertical distance between a peak and a trough in the peak search algorithm when processing sensor spectra. Must be adjusted so that all sensor peaks and no others are found. Either one value is set for all ports, or a specific value can be set for each port. |
|  |  |  |  | Peak height for reference lines. | 2 | FLOAT64 | O | - | M | The minimum vertical distance between a peak and a trough in the peak search algorithm for the reference spectrum. Must be adjusted so that not too many false peaks are found. More noise means more false peaks which means a higher value for this parameter should be set. |
|  |  |  |  | Frequency delta. | 3 | FLOAT64 | O | - | M | The maximum distance from a found absorption line in which to look for tabulated lines. For an absorption line with WistCal frequency f, the table will be searched for matching lines within the interval f+-deltaFreq. |
|  |  |  |  | Filter constant. | 4 | FLOAT64 | O | - | M | The constant by which new spectra are weighted in the low-pass filtering of calibration spectra: spectrum(t) = filterConstant * newspectrum + (1-filterConstant) * spectrum(t-1) |
|  |  |  |  | White Light min | 5 | FLOAT64 | 5 | 5 |  | White light reference values below this limit will be truncated to this limit |
|  |  |  |  | Interferometer start amplitude | 6 | FLOAT64 | 5 | 5 |  |  |
|  |  |  |  | Interferometer min step | 7 | U32 | 5 | 5 |  |  |
|  |  |  |  | Reference averages | 8 | U32 | 5 | 5 |  | Number of averages to use when pulse period has changed (due to temperature change) |
|  |  |  |  | Interferometer dispersion compensation | 9 | FLOAT64 | 5 | 5 |  | Compensates for dispersion of reference interferometer. This will create a second order dispersion term that is normalized. Typical value is 0.3 |
|  |  |  |  | Interferometer dispersion compensation 3rd order | 10 | FLOAT64 | 5 | 5 |  | Compensates for 3rd oder dispersion of reference interferometer. Typical value is 0.005 (correct value TBD) |
|  |  |  |  | Peak interpolation using second order fit | 11 | U8 | 5 | 5 |  | If 1, the points inside FWHM will be fitted to a second order polynomial using a least square fit and the peak frequency is calculated as the maximum of the fit. |
|  |  |  |  | Port threshold | 101.. 150 | FLOAT64 | 5 | 5 |  | Threshold to use for port 1 to 50 |
| Ports for sensor and calibration spectra. | “PORT” | 5 | 5 | Sensor ports | 1 | [UINT8] | O | - | M | The sensor ports. Many ports can be set at once: Set wsns port 1 2 3 4 sets ports 3 and 4 as sensor ports |
|  |  |  |  | Reference port | 2 | UINT8 | O | - | M | Absolute gas cell reference calibration port |
|  |  |  |  | Interferometer port 1 | 3 | UINT8 | O | - | M | 1st interferometer calibration port |
|  |  |  |  | Interferometer port 2 | 4 | UINT8 | O | - | M | 2nd interferometer calibration port |
| Enables interrogator function (WistSense= | “ENAB” | 5 | 5 | Enable | 1 | UINT8 | M | - | M | enable(1)/disable(0) wistsens spectrum processing.Enablink will switch of OPM |
| Enable | “FILT” | ? | 5? | Enable | 1 | UINT8 | M | - | M | enable(1)/disable(0) low pass software bassed filtering for wsns operations. Use considerable CPU. Preferable use spec filt instead |
| Returns raw FPGA optical spectra and a debugging information. Useful for testing and trimming. | ”RAWB” |  |  | Spectrum | 1 | 14336*I16 | - | - | M | Returns raw ADC spectra optionally corrected for bias and 100 MHz (Correction not working) |
|  |  |  |  | Port | 100 | U8 | M | - | M | Which port to return data for |
|  |  |  |  | Enable bias and 100 MHz correction | 101 |  | O |  |  | No effect currently |
|  |  |  |  | Seq | 150 | U32 |  |  | M | Seq number of spectra |
|  |  |  |  | MVC | 151 | U32 |  |  | M | Averages used for spectra |
|  |  |  |  | Bias | 152 | U32 |  |  | M | FPGA bias value (not used) |
|  |  |  |  | Timestamp | 153 | FLOAT64 |  |  | M | TimeS tamp. Localtime added by driver |
|  |  |  |  | Samples | 154 | UINT32 |  |  | M | Samples settings at time of interrupt |
|  |  |  |  | SPI channel | 155 | UINT16 |  |  | M | SPI channel used at interrupt (switch control), |
|  |  |  |  | SPI message | 156 | [UINT8] with length in header |  |  | M | Message to swtich over SPI, typical none but will have value before switching to another subswitch |
|  |  |  |  | Valid | 157 | UINT32 |  |  | M | If the data is valid |
|  |  |  |  | irq_rp | 160 | UINT32 |  |  | M | Ring buffer read pointer at time of interrupt |
|  |  |  |  | irg_wp | 161 | UINT32 |  |  | M | Ring buffer write pointer at time of interrupt |
|  |  |  |  | rd_rp | 162 | UINT32 |  |  | M | Ring buffer read pointer at time of read |
|  |  |  |  | rd_wp | 163 | UINT32 |  |  | M | Ring buffer write pointer at time of read |
|  |  |  |  | ring_ovfl | 164 | UINT32 |  |  | M | Count of ring buffer overflows |
|  |  |  |  | empty_irq | 165 | UINT32 |  |  | M | Count of interrupt without valid data |
|  |  |  |  | false_irq | 166 | UINT32 |  |  | M | Count of interrupt with DMA written==0 |
|  |  |  |  | tm_irq | 167 | FLOAT64 |  |  | M | Time of day for interrupt |
|  |  |  |  | tm_read | 168 | FLOAT64 |  |  | M | Time of day for read |
|  |  |  |  | rp_ptr_coll | 169 | UINT32 |  |  | M | Count of read pointer collisions |
|  |  |  |  | Current buffer | 170 | UINT32 |  |  | M | Current buffer, no value from driver? |
|  |  |  |  | FPGA time | 171 | UINT32 |  |  | M | FPGA internal time counter, no value from driver? |

big-endian     adj.     [common; From Swift's "Gulliver's Travels" via the famous paper "On Holy Wars and a Plea for Peace" by Danny Cohen, USC/ISI IEN 137, dated April 1, 1980. http://www.op.net/docs/RFCs/ien-137] Describes a computer architecture in which, within a given multi-byte numeric representation, the most significant byte has the lowest address (the word is stored `big-end-first'). Most processors, including the IBM 370 family, the PDP-10, the Motorola microprocessor families, and most of the various RISC designs are big-endian. Big-endian byte order is also sometimes called `network order'.

1 Exactly one of these data fields must be specified in the set command.

Alarms can be sent spontaneously if subscribed to. They are then sent with the structure defined in 3.2.6.

This operation returns all alarm data in one consecutive string. Tag 1, Alarm id, is used to indicate new alarm. This means that all data, in any order, after an alarm id tag until next alarm id tag or end of data belongs to that alarm.